- HYBRID OFF-GRID SYSTEM INPUTS-LOS ANGELES

| Parameters | Value | Ref. |
| --- | --- | --- |
| Project life time | 25 | - |
| Nominal discount rate | 4.5% | [1] |
| Expected inflation rate | 2% | [2] |
| Tax rate (US national average) | 5.8% | [3] |
| Maximum loss of power supply probability | 1.1% | - |
| Minimum Renewable Energy Capacity | 75% | - |
| Installation cost (per kW) | 305.57 $ | [3] |
| Overhead cost (per kW) | 549.87 $ | [3] |
| Permitting and Inspection (per kW) | 205.95 $ | [3] |
| Electrical BoS (per kW) | 399.11$ | [3] |
| Structural BoS (per kW) | 89.90 $ | [3] |
| PV Capital cost per kW | 314.18 $ | [3] |
| Replacement Cost of PV modules per kW | 314.18 $ | [3] |
| PV O&M cost ($/year/kW) | 29.49 $ | [3] |
| PV life time | 25 | - |
| DG Capital cost per kW | 308.64 $ | [4] |
| Replacement Cost of DG modules per KW | 308.64 $ | [4] |
| DG life time | 24000 hours | [5] |
| Diesel fuel cost per liter | 1.39 $ | [6] |
| DG O&M+ running cost ($/op.h) | 0.064 $ | [5] |
| BT Capital cost per kW | 234.56 $ | [3] |
| Replacement Cost of BT per kWh | 234.56 $ | [3] |
| BT life time | 15 years (Li ion) | [7] |
| Inverter Capital cost per kW | 229.38 $ | [3] |
| Replacement Cost of Inverter modules Per kW | 229.38 $ | [3] |
| Inverter life time | 25 | - |
| Charger Capital cost | 149.99 $ | [8] |
| Charger Replacement Cost | 149.99 $ | [8] |
| PV tilt angle in Los Angles | 19 | [9] |

- UTILITY RATES-GRID [10]

Base Charges:

$2.30/month (tier 1)

$7.90/month (tier 2)

$22.70/month (tier 3)

First tier rates:

$0.19488/kWh (Jan - Mar)

$0.19375/kWh (Apr - May)

$0.19375/kWh (Jun)

$0.18179/kWh (Jul - Sep)

$0.19192/kWh (Oct - Dec)

Second tier rates:

$0.25347/kWh (Jan - Mar)

$0.25234/kWh (Apr - May)

$0.25234/kWh (Jun)

$0.24038/kWh (Jul - Sep)

$0.25051/kWh (Oct - Dec)

Third tier rates:

$0.25347/kWh (Jan - Mar)

$0.25234/kWh (Apr - May)

$0.33935/kWh (Jun)

$0.32739/kWh (Jul - Sep)

$0.25051/kWh (Oct - Dec)

- RESULTS

- Off-grid system

System Size

Cpv  (kW) = 7

Cwt  (kW) = 0

Cbat (kWh) = 23

Cdg  (kW) = 1

Cinverter (kW) = 1.9521

Result:

NPC  = 33674.5877 $

LCOE  = 0.23153 $/kWh

Operation Cost  = 716.258 $

Initial Cost  = 20403.686 $

RE  = 91.065 %

Total operation and maintenance cost  = 4047.8469 $

PV Power  = 13053.2685 kWh

WT Power  = 0 kWh

DG Power  = 701.381 kWh

LPSP  = 1.0139 %

Excess Electricity = 3225.3793 kWh

Total Money paid by the user= 33674.5877 $

Total fuel consumed by DG   = 229.493 (kg/year)

Fig.1.Energy distribution in the hybrid system throughout the year

Fig.2.Sate of charge of battery throughout the year

Fig.3.Cash flow of project

Fig.4.Results for 181th day of year

Fig.5.Results for 2th day of year

Fig.6.Convergance curve for optimization

- Only Grid connection

NPC  = 38332.3709 $

LCOE  = 0.26088 $/kWh

Operation Cost  = 2068.8772 $

Fig.7.Color map of electricity prices throughout the year

Fig.8.Color table of electricity prices throughout the year

REFERENCES

[1]	“US Discount Rate.” https://ycharts.com/indicators/us_discount_rate#:~:text=US%20Discount%20Rate%20is%20at,long%20term%20average%20of%201.92%25. (accessed Jan. 13, 2023).

[2]	“Projected U.S. inflation rate 2010-2027 | Statista.” https://www.statista.com/statistics/244983/projected-inflation-rate-in-the-united-states/ (accessed Jan. 13, 2023).

[3]	V. Ramasamy et al., “US Solar Photovoltaic System and Energy Storage Cost Benchmarks, With Minimum Sustainable Price Analysis: Q1 2022,” National Renewable Energy Lab.(NREL), Golden, CO (United States), 2022.

[4]	“iGen2200 Inverter Generator.” https://westinghouseoutdoorpower.com/products/igen2200-inverter-generator (accessed Jan. 13, 2023).

[5]	A. L. Bukar, C. W. Tan, L. K. Yiew, R. Ayop, and W.-S. Tan, “A rule-based energy management scheme for long-term optimal capacity planning of grid-independent microgrid optimized by multi-objective grasshopper optimization algorithm,” Energy Convers. Manag., vol. 221, p. 113161, Oct. 2020, doi: 10.1016/j.enconman.2020.113161.

[6]	“Gasoline and Diesel Fuel Update.” https://www.eia.gov/petroleum/gasdiesel/index.php (accessed Jan. 13, 2023).

[7]	N. Narayan, T. Papakosta, V. Vega-Garita, J. Popovic-Gerber, P. Bauer, and M. Zeman, “A simple methodology for estimating battery lifetimes in Solar Home System design,” in 2017 IEEE AFRICON, Sep. 2017, pp. 1195–1201. doi: 10.1109/AFRCON.2017.8095652.

[8]	“Amazon.com : BougeRV 40A MPPT Solar Charge Controller, w/ Remote APP Control, Backlit LCD Display, Temperature Sensor, 12V/24V/36V/48V Battery Regulator, for Lithium, SLD, AGM, FLD, Gel Battery, Negative Ground : Patio, Lawn & Garden.” https://www.amazon.com/BougeRV-Controller-Temperature-Regulator-Negative/dp/B09JZGBDPB (accessed Jan. 13, 2023).

[9]	“Best Solar Panel Angle: How Do You Find It — And Does it Matter?” https://www.solarreviews.com/blog/best-solar-panel-angle (accessed Jan. 13, 2023).

[10]	“Los Angeles Utility Rates.” [Online]. Available: https://www.cdtfa.ca.gov/taxes-and-fees/rates.aspx