SAMA

Solar Alone Multi-Objective Advisor V2

Comprehensive User Documentation | v1

Free Appropriate Sustainability Technology (FAST) Research Group | Western University, London, Ontario, Canada

Contents

0. Before You Start: Installing Python	2

0.1 Checking if Python is Already Installed	2

0.2 Installing Python on Windows	2

0.3 Installing Python on macOS	2

0.4 Opening a Terminal	2

0.5 Verifying pip	2

1. Overview	3

1.1 What SAMA Optimizes	3

1.2 Key Capabilities	3

1.3 Comparison to HOMER Pro	4

2. Three Ways to Use SAMA	5

3. Method 1 - Using the Raw Python Code	5

3.1 Prerequisites	5

3.2 Downloading and Opening the Code	5

3.3 Configuring Parameters Directly in Input_Data.py	6

3.3.1 System Components	6

3.3.2 Optimization Search Space	6

3.3.3 Load Input	6

3.3.4 Weather Input	7

3.3.5 Economic Parameters	8

3.3.6 Component Cost Parameters (Bottom-Up, Pricing_method = 2)	8

3.3.7 Electricity Tariff Structure	8

3.3.8 Natural Gas Tariff (for Heat Pump)	9

3.3.9 System Constraints	9

3.4 Running the Optimization	10

3.4.1 Using run_sama_optimized.py (recommended for raw-code users)	10

3.4.2 Running optimizer modules directly	10

3.4.3 Using the examples/ folder	10

4. Method 2 - Using the Python Package (pip install sama)	11

4.1 Installation	11

4.2 Step 1 - Run the Configuration Wizard	11

4.3 Step 2 - Run the Optimization	12

4.3.1 sama-run command options	12

4.4 Step 3 - View Results	13

4.4.1 Chart files generated in sama_outputs/figs/	13

4.4.2 Data files generated in sama_outputs/data/	14

4.5 Using the Python API	14

4.5.1 Basic API workflow	14

4.5.2 Available optimizer classes	15

4.5.3 Config loader helper functions	15

4.5.4 Important: Package Name vs Import Name	15

4.6 Using Your Own Custom Data Files	15

4.7 Typical Workflow for a New Project Location	16

4.8 Understanding the Working Directory	16

4.9 What to Expect on the First Run	16

5. Method 3 - Windows .exe Application (Alpha)	17

6. The Configuration File (sama_config_COMPLETE.yaml)	18

6.1 File Structure Overview	18

6.2 Key Parameter Reference	18

7. Optimization Algorithms	22

7.1 Particle Swarm Optimization (PSO)	22

7.2 Advanced Differential Evolution (ADE)	22

7.3 Improved Artificial Bee Colony (ABC)	22

7.4 Grey Wolf Optimizer (GWO)	23

7.5 Choosing an Algorithm	23

8. Energy Management System (EMS)	24

8.1 Base Dispatch (EMS.py)	24

8.2 EV Smart Charging and V2X (EMS_EV.py)	24

8.3 Heat Pump Energy Attribution (EMS_HP.py)	25

9. Component Models	25

9.1 Solar PV	25

9.2 Wind Turbine	25

9.3 Battery Storage	25

9.4 Diesel Generator	26

9.5 Heat Pump Models	26

10. Financial Model	27

10.1 Net Present Cost (NPC)	27

10.2 Levelized Cost of Energy (LCOE)	27

10.3 Levelized Emissions (LEM)	27

10.4 25-Year Cash Flow Projection	27

11. Bundled Data Files (content/ folder)	27

12. Input Data Formats	28

12.1 Electrical Load CSV (Eload.csv)	28

12.2 METEO.csv (SAM NSRDB format)	28

12.3 Plane-of-Array Irradiance CSV (Irradiance.csv)	29

12.4 house_load.xlsx (Thermal Load)	29

13. Troubleshooting	30

14. Quick Reference - Minimum Working Example	31

15. Package Module Structure	32

16. License and Citation	34

16.1 License	34

16.2 Authors	34

16.3 Citing SAMA	34

17. Technical Framework and Mathematical Formulation	34

17.1 Particle Swarm Optimization Algorithm	34

17.2 Solar PV Power Output Model	35

17.3 Battery Storage Model (KiBaM)	35

17.4 Backup (Diesel or Gasoline) Generator Model	36

17.5 Wind Turbine Model	36

17.6 Energy Management Strategy (EMS)	36

17.7 Economic Model	37

17.8 Validation Against HOMER Pro	38

18. References	39

19. SAMA Publications and Further Reading	41

Appendix A: Where to Find Input Data for SAMA	42

A.1 Financial and Economic Parameters	42

A.2 System Constraints	42

A.3 Weather and Meteorological Data (METEO.csv)	42

A.4 Electrical Load Data (Eload.csv)	43

A.5 Thermal Load Data (house_load.xlsx) for Heat Pump	43

A.6 Solar PV Parameters	43

A.7 Wind Turbine Parameters	43

A.8 Battery Storage Parameters	43

A.9 Diesel Generator Parameters	44

A.10 Heat Pump Parameters	44

A.11 Electric Vehicle Parameters	44

A.12 Grid and Utility Rate Parameters	44

A.13 Summary of Recommended Data Sources	45

# 0. Before You Start: Installing Python

SAMA requires Python 3.9 or higher. If you do not already have Python installed on your computer, follow the steps below before proceeding to any other section of this guide.

## 0.1 Checking if Python is Already Installed

Open a terminal (Command Prompt on Windows, Terminal on macOS/Linux) and type:

python --version

If you see a version number such as Python 3.11.4, Python is already installed and you can skip this section. If you see an error or a version below 3.9, follow the steps below.

## 0.2 Installing Python on Windows

Go to https://www.python.org/downloads/ and click Download Python (choose the latest 3.x version).

Run the installer. On the first screen, check the box that says "Add Python to PATH" before clicking Install Now. This is the most important step - without it, Python commands will not work in the terminal.

Once installation is complete, close and reopen your Command Prompt and verify with python --version

## 0.3 Installing Python on macOS

Go to https://www.python.org/downloads/ and download the macOS installer.

Run the .pkg installer and follow the on-screen steps.

Open Terminal and verify with: python3 --version

## 0.4 Opening a Terminal

Throughout this guide, instructions refer to running commands in a terminal. Here is how to open one on each platform:

Windows: Press the Windows key, type cmd, and press Enter. Or open File Explorer, navigate to your project folder, click the address bar, type cmd, and press Enter to open a terminal directly in that folder.

macOS: Press Command + Space, type Terminal, and press Enter.

Linux: Press Ctrl + Alt + T.

## 0.5 Verifying pip

pip is the Python package installer and is included automatically with Python 3.9 and later. Verify it is available:

pip --version

If pip is not found, run: python -m ensurepip –upgrade

# 1. Overview

SAMA (Solar Alone Multi-Objective Advisor) is an open-source Python platform for the design, sizing, and operation optimization of hybrid renewable energy microgrids. It performs location-specific techno-economic assessment using site-dependent electrical and thermal load profiles, meteorological data, component degradation models, and utility-specific billing structures.

Unlike proprietary tools such as HOMER Pro and HOMER Grid, which are now closed-source and license-restricted, SAMA provides transparent optimization algorithms, fully extensible architecture, and no cost barriers for researchers, practitioners, and resource-constrained institutions.

## 1.1 What SAMA Optimizes

SAMA finds the optimal combination of five design variables:

- Number of PV modules (Npv) in units of rated PV capacity (kW)

- Number of wind turbines (Nwt)

- Number of battery packs (Nbat)

- Number (size) of diesel generator units (N_DG)

- Inverter capacity (Cn_I) in kW

The search space is bounded by VarMin and VarMax (default: 0 to 60 for each variable). The optimizer explores this space to minimize the primary objective, Net Present Cost (NPC), while satisfying reliability, renewable fraction, budget, and rooftop-area constraints. When EM is set to 1, emissions (Levelized Emissions, LEM) are included as a second objective.

## 1.2 Key Capabilities

| Capability | Detail |
| --- | --- |
| System components | Solar PV, wind turbines, battery storage (lead-acid or lithium-ion), diesel generator, utility grid, air-source heat pump (Bosch or Goodman), electric vehicle (V2X) |
| Optimization algorithms | Particle Swarm Optimization (PSO), Advanced Differential Evolution (ADE), Artificial Bee Colony (ABC), Grey Wolf Optimizer (GWO) |
| Energy management | 8,760-hour hourly dispatch with EMS, EV smart charging/discharging with V2X lookahead arbitrage, heat pump thermal-electric coupling |
| Electricity tariffs | 8 rate structures: flat, seasonal, monthly, tiered, seasonal tiered, monthly tiered, Time-of-Use (TOU), Ultra-Low TOU (ULO) |
| Natural gas tariffs | 8 rate structures for NG: flat, seasonal, monthly, tiered, seasonal tiered, monthly tiered, PG&E G-1 therms-based, Enbridge EGD 4-tier m3-based |
| Financial analysis | 25-year Net Present Cost, LCOE, levelized emissions, escalation rates, RE incentives (e.g. 30% federal tax credit), net metering reconciliation |
| Load inputs | 10 electrical load input modes (CSV, monthly/annual averages, generic profiles), thermal load from Excel |
| Weather inputs | SAM NSRDB API integration, user CSV files, monthly/annual averages |
| Outputs | Convergence plots, cash flow charts, energy distribution, battery SOC, grid interconnection, EV energy charts, HP performance charts, hourly data CSV |
| License | GNU General Public License v3.0 (GPL-3.0), free to use, modify, and distribute |

## 1.3 Comparison to HOMER Pro

SAMA has been validated against HOMER Pro in diverse climatic zones (Sacramento, California and New Bern, North Carolina) and achieves close agreement in optimal system configurations and performance metrics. SAMA additionally supports constraint scenarios such as rooftop area limits, net-metering caps, renewable fraction requirements, and loss-of-power-supply probability, which are handled more flexibly than in proprietary tools.

# 2. Three Ways to Use SAMA

SAMA can be used in three different ways depending on your background and needs:

| Method | Who It Is For | How to Get It |
| --- | --- | --- |
| Method 1: Raw Python Code | Researchers, developers, and advanced users who want to modify algorithms or parameters directly in the source code | Download from GitHub and open in a Python IDE such as PyCharm |
| Method 2: Python Package (pip) | Users who want to run SAMA as a command-line tool or use it in their own Python scripts without modifying internals | pip install sama (available as the SAMA Py Package) |
| Method 3: Windows .exe Application (Alpha) | Non-technical users who want a point-and-click graphical user interface without installing Python. Note: this version is currently in the alpha development phase. | Available on request from the developers |

# 3. Method 1 - Using the Raw Python Code

This method gives you full access to all source files. You can directly edit Input_Data.py to set your parameters and run the optimizer scripts directly. This is the approach used in the examples/ folder.

## 3.1 Prerequisites

- Python 3.9 or higher

- A Python IDE such as PyCharm (recommended), VS Code, or Spyder

- Git (optional, for cloning the repository) or a browser to download the ZIP from GitHub

## 3.2 Downloading and Opening the Code

The SAMA v2.0.1 source code is hosted on GitHub at the following location:

GitHub Repository (Method 1 source code): https://github.com/Sas1997/SAMA/tree/main/Backend%20Codes/SAMA%20V2.0.1-GitHub

- Open the URL above in your browser.

- Click the green Code button and choose Download ZIP, or clone with Git:

git clone https://github.com/Sas1997/SAMA.git

- Extract the ZIP (or navigate into the cloned folder) and open the folder Backend Codes/SAMA V2.0.1-GitHub in PyCharm (or your preferred IDE).

- PyCharm will detect the project structure automatically. Set up a Python interpreter (Python 3.9 or higher) in File > Settings > Project > Python Interpreter.

- Install the required packages. You can do this in the PyCharm terminal or any command prompt:

pip install numpy pandas scipy numba matplotlib openpyxl seaborn numpy-financial questionary PyYAML

- Verify the setup by running:

python -c "from sama.core.Input_Data import InData; print('OK')"

## 3.3 Configuring Parameters Directly in Input_Data.py

Open sama/sama/core/Input_Data.py in your IDE. All parameters are set in the Input_Data.__init__ method. The key sections to edit are described below.

### 3.3.1 System Components

Enable or disable each component by setting its flag to 1 (included) or 0 (excluded):

self.PV = 1       # Solar PV

self.WT = 0       # Wind turbine

self.DG = 1       # Diesel generator

self.Bat = 1      # Battery storage

self.Grid = 1     # Utility grid connection

self.HP = 1       # Heat pump (Goodman or Bosch)

self.EV = 0       # Electric vehicle

self.Lead_acid = 0  # Lead-acid battery

self.Li_ion = 1     # Li-ion battery

### 3.3.2 Optimization Search Space

VarMin and VarMax define the lower and upper bounds for the five design variables [Npv, Nwt, Nbat, N_DG, Cn_I]:

self.VarMin = np.array([0, 0, 0, 0, 0])      # lower bounds

self.VarMax = np.array([60, 60, 60, 20, 60]) # upper bounds

MaxIt, nPop, and Run_Time control the optimizer:

self.MaxIt = 200    # maximum number of iterations

self.nPop = 50      # population size (number of candidate solutions)

self.Run_Time = 1   # number of independent optimization runs

### 3.3.3 Load Input

Set load_type to choose how the electrical load is provided:

| load_type value | Description | Required variable |
| --- | --- | --- |
| 1 | Hourly load from a CSV file (8,760 rows) | path_Eload: path to CSV |
| 2 | Monthly hourly average load | Monthly_haverage_load: array of 12 values (kW) |
| 3 | Monthly daily average load | Monthly_daverage_load: array of 12 values (kWh/day) |
| 4 | Monthly total load | Monthly_total_load: array of 12 values (kWh/month) |
| 5 | Generic profile scaled to monthly totals | user_defined_load: array of 12 monthly totals |
| 6 | Annual hourly average | Annual_haverage_load: single value (kW) |
| 7 | Annual daily average | Annual_daverage_load: single value (kWh/day) |
| 8 | Generic profile scaled to annual total | Annual_total_load: single value (kWh/year) |
| 9 | Exact generic load profile (unscaled) | peak_month: 'July' or 'January' |
| 10 | Generic profile scaled to user daily profile from CSV | path_Eload_daily: path to 24-row CSV |

Note: The content/ folder contains a sample Eload.csv for Boston, MA. To use your own load, replace this file or set path_Eload to your file path.

### 3.3.4 Weather Input

G_type selects irradiance input, T_type selects temperature, and WS_type selects wind speed:

| Type flag | Value 1 | Value 2 | Value 3 | Value 4 |
| --- | --- | --- | --- | --- |
| G_type (irradiance) | From NSRDB via SAM API (uses METEO.csv) | User CSV file (plane-of-array) | N/A | N/A |
| T_type (temperature) | From NSRDB via SAM API (uses METEO.csv) | User CSV file | Monthly averages (12-value array) | Annual average (single value) |
| WS_type (wind speed) | From NSRDB via SAM API (uses METEO.csv) | User CSV file | Monthly averages (12-value array) | Annual average (single value) |

The default weather_url points to the bundled METEO.csv, which contains sample data for London, Ontario from the National Solar Radiation Database. Key PV geometry parameters:

azimuth = 180   # degrees (180 = south-facing)

tilt    = 33    # degrees (tilt angle of PV modules)

soiling = 5     # percent soiling losses

### 3.3.5 Economic Parameters

| Parameter | Default | Description |
| --- | --- | --- |
| n | 25 | Project lifetime in years |
| n_ir_rate | 2.75 | Nominal discount rate (%) |
| e_ir_rate | 2.0 | Expected inflation rate (%) |
| Budget | 2,000,000 | Maximum allowable capital cost ($) |
| Tax_rate | 0 | Equipment sales tax (%) |
| RE_incentives_rate | 30 | Renewable energy incentives / federal tax credit (%) |
| Pricing_method | 2 | 1 = top-down (total system $/kW), 2 = bottom-up (itemized costs) |

### 3.3.6 Component Cost Parameters (Bottom-Up, Pricing_method = 2)

| Component | Capital cost param | Replacement param | O&M param | Units |
| --- | --- | --- | --- | --- |
| PV modules | C_PV = 338 | R_PV = 338 | MO_PV = 30.36 | $/kW, $/kW, $/kW/year |
| Inverter | C_I = 314 | R_I = 314 | MO_I = 0 | $/kW, $/kW, $/kW/year |
| Wind turbine | C_WT = 1200 | R_WT = 1200 | MO_WT = 40 | $/kW, $/kW, $/kW/year |
| Diesel generator | C_DG = 818 | R_DG = 818 | MO_DG = 0.016 | $/kW, $/kW, $/op.h |
| Battery | C_B = 1450 | R_B = 1450 | MO_B = 10 | $/kWh, $/kWh, $/kWh/year |
| Heat pump | C_HP = 109.5 | R_HP = 109.5 | MO_HP = 20 | $/1000 BTU/hr, $/1000 BTU/hr, $/year |
| EV battery replacement | N/A | R_EVB = 27000 | MO_EV = 0 | $, $/year |

Additional PV engineering costs (bottom-up) are summed into Engineering_Costs ($/kW):

self.Fieldwork = 178         # $/kW

self.Officework = 696        # $/kW

self.Electrical_BoS = 333    # $/kW

self.Structrual_BoS = 237    # $/kW

### 3.3.7 Electricity Tariff Structure

Set rateStructure to choose the utility tariff model for grid electricity purchases:

| rateStructure | Type | Required parameters |
| --- | --- | --- |
| 1 | Flat rate | flatPrice: single $/kWh value |
| 2 | Seasonal rate | seasonalPrices [summer, winter], season [12-value month array] |
| 3 | Monthly rate | monthlyPrices: 12-value array ($/kWh per month) |
| 4 | Tiered rate | tieredPrices, tierMax: tier prices and monthly kWh limits |
| 5 | Seasonal tiered rate | seasonalTieredPrices [2 seasons x 3 tiers], seasonalTierMax, season |
| 6 | Monthly tiered rate | monthlyTieredPrices [12 x 3], monthlyTierLimits [12 x 3] |
| 7 | Time-of-Use (TOU) | onPrice, midPrice, offPrice [summer, winter], onHours, midHours, season, treat_special_days_as_offpeak |
| 8 | Ultra-Low TOU (ULO) | onPrice, midPrice, offPrice, ultraLowPrice [summer, winter], onHours, midHours, ultraLowHours, season, treat_special_days_as_offpeak |

Grid sell price structure is set separately with sellStructure:

| sellStructure | Description | Parameter |
| --- | --- | --- |
| 1 | Flat sell price | Csell = np.full(8760, price): one price all year |
| 2 | Monthly sell price | monthlysellprices: 12-value array ($/kWh per month) |
| 3 | Same as buy price | Csell = Cbuy: NEM at retail rate |

Net metering is controlled by NEM (1 = enabled) and NEM_fee (one-time connection fee in $). When NEM = 1, surplus generation credits are reconciled annually.

### 3.3.8 Natural Gas Tariff (for Heat Pump)

When HP = 1 and a natural gas furnace baseline is being compared, set rateStructure_NG to one of 8 structures analogous to the electricity rate structures. The NG price is entered in the user's native unit ($/m3, $/therm, or $/kWh) and SAMA converts internally using NG_energycontent (10.81 kWh/m3) and Furnace_eff (0.94).

### 3.3.9 System Constraints

| Parameter | Default | Description |
| --- | --- | --- |
| LPSP_max_rate | 0.000999 | Maximum Loss of Power Supply Probability (LPSP) as a percentage. The system must meet at least (100 minus LPSP_max_rate)% of demand. |
| RE_min_rate | 50 | Minimum renewable energy fraction required (%). System must have at least this fraction from PV and WT. |
| EM | 0 | Objective mode. 0 = minimize NPC only. 1 = minimize NPC + Levelized Emissions (LEM). |
| Budget | 2,000,000 | Maximum total capital cost ($). Infeasible solutions above this are penalized. |
| cap_option | 2 | Capacity sizing constraint. 1 = explicit cap in kW (cap_size). 2 = size to annual load. 3 = rooftop area limit. 4 = no limit. |

## 3.4 Running the Optimization

After configuring Input_Data.py, run one of the optimizer scripts directly. In PyCharm, right-click the script and choose Run, or use the terminal.

### 3.4.1 Using run_sama_optimized.py (recommended for raw-code users)

python run_sama_optimized.py --algorithm pso

python run_sama_optimized.py --algorithm ade

python run_sama_optimized.py --algorithm abc

python run_sama_optimized.py --algorithm gwo

This script loads sama_config_COMPLETE_HYBRID.yaml (if present) and applies it to InData at runtime, then runs the chosen algorithm.

### 3.4.2 Running optimizer modules directly

python sama/optimizers/pso.py        # Particle Swarm Optimization

python sama/optimizers/ade.py        # Advanced Differential Evolution

python sama/optimizers/run_abc.py    # Artificial Bee Colony

python sama/optimizers/gwo.py        # Grey Wolf Optimizer

### 3.4.3 Using the examples/ folder

The examples/optimization_case.py file contains a complete working example with all 8 electricity rate structures and all 6 natural gas rate structures fully commented. It defines a Config class with all parameters and runs the full optimization workflow.

python examples/optimization_case.py

# 4. Method 2 - Using the Python Package (pip install sama)

The SAMA pip package (SAMA Py Package) is available through the Python Package Index (PyPI). It provides two console commands, sama-config and sama-run, that guide you through configuration and run the optimization without editing any source files. This is the recommended approach for most users.

## 4.1 Installation

- Install SAMA from PyPI (the SAMA Py Package):

pip install sama

- Verify the installation:

sama-config --help

sama-run --help

Note: Python 3.9 or higher is required. On Windows, ensure Python is in your PATH. All dependencies (numpy, pandas, scipy, numba, matplotlib, openpyxl, seaborn, numpy-financial, questionary, PyYAML) are installed automatically.

## 4.2 Step 1 - Run the Configuration Wizard

In your working directory (the folder where you want results to be saved), run:

sama-config

The wizard launches an interactive terminal interface with 18 sections that guide you through every parameter category. You answer questions using your keyboard; arrow keys to select and Enter to confirm. At the end of each section the wizard saves progress and moves to the next.

| Wizard Section | Parameters Covered |
| --- | --- |
| Section 1: System Components | Which components to include: PV, WT, DG, Battery (lead-acid or Li-ion), Grid, Heat Pump (Bosch or Goodman), EV |
| Section 2: Optimization Settings | Algorithm (PSO/ADE/ABC/GWO), population size, iterations, run count, design variable bounds (VarMin, VarMax) |
| Section 3: Project and Calendar | Project lifetime (n), simulation year, public holiday list |
| Section 4: Electrical Load | Load type selection (1-10), CSV file paths or manual values, previous-year load for NEM calculations |
| Section 5: Thermal Load | Heat pump thermal load source (house_load.xlsx or manual) |
| Section 6: Weather and Irradiance | METEO.csv path or manual weather input, PV tilt/azimuth/soiling |
| Section 7: Constraints | LPSP maximum, renewable energy minimum fraction, budget limit, capacity constraint type |
| Section 8: Financial | Discount rates, inflation rate, tax rate, RE incentives/federal tax credit |
| Section 9: PV and Inverter | Pricing method, module efficiency, NOCT parameters, lifetimes, capital/replacement/O&M costs |
| Section 10: Wind Turbine | Hub height, cut-in/rated/cut-out speeds, power law coefficient, costs |
| Section 11: Diesel Generator | Rated capacity, fuel curve (a, b), minimum load ratio, emissions, fuel cost, lifetime |
| Section 12: Battery Storage | Li-ion or lead-acid selection, SOC limits, round-trip efficiency, capacity, lifetime throughput, costs |
| Section 13: Heat Pump | HP brand (Bosch or Goodman), rated size, lifetime, costs |
| Section 14: Electric Vehicle | Battery capacity, SOC limits, charging power, range, daily trip, departure/arrival times, degradation |
| Section 15: Grid and NEM | Net metering flag, capacity option (cap_size), grid tax, annual service charge, sell structure |
| Section 16: Electricity Rate Structure | Rate type (1-8), prices, TOU hours, seasons, holidays treatment |
| Section 17: Natural Gas Rate Structure | NG unit (m3/therms/kWh), rate type (1-8), NG prices, escalation |
| Section 18: Output Settings | Output directory, compare-with-grid scenario flag |

When the wizard completes, it saves a file called sama_config_COMPLETE.yaml in your working directory. This YAML file contains all your parameters and can be edited manually or re-run through the wizard at any time.

## 4.3 Step 2 - Run the Optimization

With sama_config_COMPLETE.yaml in your working directory, run:

sama-run

SAMA will automatically find the config file, load all parameters, and run the optimization algorithm specified in the config (optimization_algorithm key). Additional options are available:

### 4.3.1 sama-run command options

| Option | Description | Example |
| --- | --- | --- |
| -c / --config | Path to config YAML (default: sama_config_COMPLETE.yaml in cwd) | sama-run -c my_project.yaml |
| -a / --algorithm | Override the algorithm from the config file | sama-run -a pso |
| --output | Override the output directory from the config | sama-run --output results/run1 |
| --dry-run | Validate the config file without running optimization | sama-run --dry-run --verbose |
| --no-gui | Disable matplotlib GUI (use Agg backend for servers) | sama-run --no-gui |
| --verbose | Show full parameter log and config summary | sama-run --verbose |

The optimization_algorithm key in the YAML accepts: pso, ade, abc, or gwo.

## 4.4 Step 3 - View Results

Results are saved to the output directory specified in the config (default: sama_outputs/ in your working directory). The folder is organized as:

sama_outputs/

Optimization.png    convergence curve (best cost vs. iteration)

figs/               all chart PNG/SVG files

data/               CSV data files

### 4.4.1 Chart files generated in sama_outputs/figs/

| File | Description |
| --- | --- |
| Cash_Flow.svg | 25-year cash flow breakdown (capital, replacement, O&M, fuel, grid) |
| Cash Flow_ADV.png | Advanced cash flow with grid comparison (when compare_with_grid = 1) |
| Multiple_Cash_Flow_ADV.png | Multi-run cash flow comparison (when Run_Time > 1) |
| Energy Distribution.png | Annual energy supply breakdown by source (PV, WT, DG, battery, grid) |
| Battery State of Charge.png | Hourly battery SOC profile over the year |
| Grid Interconnection.png | Hourly grid import and export over the year |
| Specific day results.png | Detailed 24-hour power flow for a representative day |
| Daily-Monthly-Yearly average cost of energy system.png | Cost comparison across daily, monthly, and annual timescales |
| Daily-Monthly-Yearly average hourly cost of connecting to the grid.png | Hourly grid cost patterns by season |
| Daily-Monthly-Yearly average cost of only grid-connected system.png | Baseline grid-only cost for comparison |
| Grid Hourly Cost.png | Hourly grid electricity cost profile |
| Daily-Monthly-Yearly average earning Sell to the Grid.png | Revenue from grid exports (if NEM enabled) |
| Energy Balance.png | Hourly energy balance (supply vs. demand) |
| electricity_comparison.png | SAMA system vs. grid-only electricity cost comparison |
| EV Energy.png | EV charging and discharging energy over the year |
| EV Sp Results 1.png | EV-specific results chart 1 (when EV = 1) |
| EV Sp Results 2.png | EV-specific results chart 2 (when EV = 1) |
| hp_ambient_conditions.png | Ambient temperature and pressure conditions for heat pump (when HP = 1) |
| hp_heating_performance.png | Heat pump heating power and COP (when HP = 1) |
| hp_cooling_performance.png | Heat pump cooling power and COP (when HP = 1) |
| hp_cop_vs_temp.png | COP as a function of ambient temperature (when HP = 1) |
| hp_monthly_summary.png | Monthly heat pump energy and COP summary (when HP = 1) |
| Optimization.png | Optimizer convergence curve (saved in sama_outputs/ root) |

### 4.4.2 Data files generated in sama_outputs/data/

| File | Description |
| --- | --- |
| Outputforplotting.csv | Complete hourly time-series: PV output, WT output, battery SOC, DG power, grid import/export, load, EV charge/discharge, and all cost variables for all 8,760 hours |
| cash_flow_data.csv | Annual cash flow breakdown over the 25-year project lifetime |

## 4.5 Using the Python API

After pip install sama, SAMA can also be used directly in Python scripts without the command-line interface:

### 4.5.1 Basic API workflow

from sama.core.Input_Data import InData

from sama.cli.config_loader import load_config, apply_config

# Load a YAML config produced by sama-config

config = load_config('sama_config_COMPLETE.yaml')

# Apply all parameters to the InData singleton

apply_config(config)

# Run the optimizer

from sama.optimizers.swarm import Swarm

swarm = Swarm()

swarm.optimize()

### 4.5.2 Available optimizer classes

| Algorithm | Import | Class |
| --- | --- | --- |
| Particle Swarm Optimization | from sama.optimizers.swarm import Swarm | Swarm() |
| Advanced Differential Evolution | from sama.optimizers.AdvancedDifferentialEvolution import AdvancedDifferentialEvolution | AdvancedDifferentialEvolution() |
| Artificial Bee Colony | from sama.optimizers.ArtificialBeeColony import ImprovedArtificialBeeColony | ImprovedArtificialBeeColony() |
| Grey Wolf Optimizer | from sama.optimizers.GreyWolfOptimizer import GreyWolfOptimizer | GreyWolfOptimizer() |

### 4.5.3 Config loader helper functions

| Function | Description |
| --- | --- |
| load_config(path) | Load a YAML config file and return it as a Python dictionary |
| apply_config(config) | Apply a config dictionary to the InData singleton and return the fully reconfigured InData instance |
| save_config(config, path) | Save a config dictionary to a YAML file |
| merge_configs(*paths) | Merge multiple YAML config files into one dictionary (later files override earlier ones) |

## 4.5.4 Important: Package Name vs Import Name

The SAMA Python package is published on PyPI under the distribution name SAMAbyRenXera. This is the name you use with pip to install the package. However, once installed, the package is imported in Python code using the name sama (all lowercase). These are two different names for the same package:

Install with pip (use the PyPI distribution name):

pip install SAMAbyRenXera

Import in Python scripts (use the internal package name):

import sama

Command line tools installed automatically:

sama-config
sama-run

When searching for the package on the PyPI website, look for SAMAbyRenXera. When writing Python scripts that use SAMA, always write import sama.

## 4.6 Using Your Own Custom Data Files

SAMA ships with bundled default data files for London, Ontario (weather, electrical load, and heat pump data). When you run sama-config and provide your own file paths, the wizard automatically copies those files into the local sama/content/ folder in your working directory. SAMA reads from this local folder instead of the bundled defaults.

You can also copy your custom files directly into the sama/content/ folder yourself, and SAMA will use them automatically when you run sama-run. The sama/content/ folder is created in your working directory the first time you run sama-config or sama-run.

The expected file names and formats are:

Eload.csv: Hourly electrical load profile. One column, 8,760 rows, no header. Values in kW.

METEO.csv: SAM NSRDB weather file for your location. Two header rows followed by 8,760 hourly data rows. Must include columns: GHI, DNI, DHI, Temperature, Wind Speed, Pressure. Download from https://nsrdb.nrel.gov/

house_load.xlsx: Building thermal load. Excel file with heating load (Hload) in column 2 and cooling load (Cload) in column 3, 8,760 rows each, in kW.

Irradiance.csv: Only needed when G_type = 2 (manual plane-of-array irradiance). One column, 8,760 rows, no header. Values in W/m2.

If a file is not present in the local sama/content/ folder, SAMA automatically falls back to the bundled default. You only need to provide files you want to customize.

## 4.7 Typical Workflow for a New Project Location

Step 1: Create an empty project folder, for example C:\Projects\MyCity_SAMA

Step 2: Download your METEO.csv weather file from https://nsrdb.nrel.gov/ for your project location.

Step 3: Prepare your Eload.csv file with 8,760 rows of hourly electrical load data in kW.

Step 4: If modeling a heat pump, prepare house_load.xlsx with Hload and Cload columns.

Step 5: Open a terminal in your project folder and run sama-config. The wizard will ask for paths to your data files and copy them into sama/content/ automatically.

Step 6: When the wizard finishes, sama_config_COMPLETE.yaml is saved in your project folder. You can open it in any text editor to review or manually adjust settings.

Step 7: Run sama-run to start the optimization. Results are saved in sama_outputs/ inside your project folder.

## 4.8 Understanding the Working Directory

SAMA creates all its folders (sama/content/, sama_inputs/, sama_outputs/) relative to the folder where you run the sama-config and sama-run commands. This folder is called the working directory. Always run both commands from the same project folder so that the configuration file, custom data files, and results all stay together in one place.

On Windows: navigate to your project folder in File Explorer, click the address bar, type cmd, and press Enter. This opens a Command Prompt already pointed at that folder.

## 4.9 What to Expect on the First Run

The very first time you run sama-run after installation, SAMA uses Numba to compile the energy management system (EMS) functions. This compilation is a one-time process and can take several minutes. You will see messages in the terminal while this happens. Subsequent runs will be significantly faster because the compiled code is cached on your computer.

For a typical run with MaxIt=200 and nPop=50, expect the optimization itself to take between 5 and 30 minutes depending on your hardware and which components (EV, heat pump) are enabled.

# 5. Method 3 - Windows .exe Application (Alpha)

A Windows executable (.exe) version of SAMA is currently in the alpha development phase and is available on request from the developers. This version provides a graphical user interface (GUI) that does not require Python to be installed on the user's computer. It is intended for non-technical users who prefer a point-and-click experience.

Because this version is in the alpha phase, some features may be incomplete or subject to change. Users are encouraged to report any issues to the development team.

To request the Windows .exe version, contact the FAST research group at Western University. Contact information is available at the GitHub repository: https://github.com/Sas1997/SAMA

Windows .exe Application (Alpha Development Phase): The .exe version is not distributed via PyPI. It must be obtained directly from the developers. No Python installation is required to run the .exe. It uses the same algorithms and produces the same outputs as the Python package. This version is currently in the alpha development phase. Features may be incomplete.

# 6. The Configuration File (sama_config_COMPLETE.yaml)

Whether you use the wizard (sama-config) or edit it manually, the configuration file is a standard YAML text file. You can open it in any text editor. Below is a complete reference of all keys.

## 6.1 File Structure Overview

input_directory: C:\path\to\project         # working directory (used by raw code)

output_directory: C:\path\to\sama_outputs   # results output folder

optimization_algorithm: abc                # pso | ade | abc | gwo

# Optimization settings

MaxIt: 200

nPop: 50

Run_Time: 1

VarMin: [0.0, 0.0, 0.0, 0.0, 0.0]

VarMax: [60.0, 60.0, 60.0, 20.0, 60.0]

# System components (1=included, 0=excluded)

PV: 1   WT: 1   DG: 1   Bat: 1   Grid: 1   HP: 1   EV: 1

Li_ion: 1   Lead_acid: 0

NEM: 1

# Project calendar

n: 25           # project lifetime (years)

year: 2023      # simulation year

holidays: [1, 51, 97, ...]   # day-of-year numbers for public holidays

## 6.2 Key Parameter Reference

| YAML key | Type | Default | Description |
| --- | --- | --- | --- |
| Optimization |  |  |  |
| MaxIt | int | 200 | Maximum optimizer iterations |
| nPop | int | 50 | Population size (number of candidate solutions) |
| Run_Time | int | 1 | Independent optimization runs (best result kept) |
| VarMin / VarMax | list[4] | [0,0,0,0,0] / [60,60,60,20,60] | Lower/upper bounds for [Npv, Nwt, Nbat, N_DG, Cn_I] |
| Cash_Flow_adv | int | 0 | 0 = standard cash flow; 1 = advanced multi-run cash flow chart |
| Calendar |  |  |  |
| n | int | 25 | Project lifetime in years |
| year | int | 2023 | Simulation year (affects day-of-week and leap year) |
| holidays | list[int] | [1,51,97,...] | Day-of-year numbers for public holidays (affects TOU pricing) |
| Load |  |  |  |
| load_type | int | 1 | Electrical load input mode (1-10, see Section 3.3.3) |
| load_previous_year_type | int | 1 | Previous year load mode for NEM credit calculation (1-11) |
| Tload_type | int | 1 | Thermal load input type (1 = from house_load.xlsx) |
| Weather |  |  |  |
| G_type | int | 1 | Irradiance input mode (1=NSRDB/SAM, 2=CSV) |
| T_type | int | 1 | Temperature input mode (1=NSRDB/SAM, 2=CSV, 3=monthly avg, 4=annual avg) |
| WS_type | int | 1 | Wind speed input mode (1=NSRDB/SAM, 2=CSV, 3=monthly avg, 4=annual avg) |
| azimuth | float | 180.0 | PV panel azimuth angle (degrees, 180 = south-facing) |
| tilt | float | 33.0 | PV panel tilt angle (degrees) |
| soiling | float | 5.0 | PV soiling losses (%) |
| weather_url | str | METEO.csv | Path to NSRDB/SAM meteorological CSV file |
| Constraints |  |  |  |
| LPSP_max_rate | float | 0.000999 | Maximum LPSP (% of annual demand that can be unmet) |
| RE_min_rate | float | 50.0 | Minimum renewable energy fraction required (%) |
| EM | int | 0 | 0 = minimize NPC; 1 = minimize NPC + LEM (emissions) |
| Budget | float | 2,000,000 | Maximum allowable capital cost ($) |
| cap_option | int | 1 | 1=size cap (kW), 2=annual load, 3=rooftop area, 4=no limit |
| cap_size | float | 15.0 | PV system size cap in kW (used when cap_option=1) |
| Financial |  |  |  |
| n_ir_rate | float | 2.75 | Nominal discount rate (%) |
| e_ir_rate | float | 2.0 | Expected inflation rate (%) |
| Tax_rate | float | 0.0 | Equipment sales tax (%) |
| RE_incentives_rate | float | 30.0 | Federal/provincial RE incentives / tax credit (%) |
| Pricing_method | int | 2 | 1=top-down (Total_PV_price $/kW), 2=bottom-up (itemized) |
| Grid |  |  |  |
| Grid_sale_tax_rate | float | 13.0 | Electricity sales tax (%) |
| Grid_Tax_amount | float | 0.0353 | Fixed grid tax in $/kWh (e.g. delivery charge component) |
| Grid_credit | float | 0.0 | Annual grid credit to customers in $ (e.g. rebate) |
| Grid_escalation_rate | list[12] | 2.0 per year | Annual electricity price escalation rate per year (%) |
| NEM | int | 1 | 1 = net metering enabled (annual credit reconciliation) |
| NEM_fee | float | 0.0 | One-time NEM interconnection setup fee ($) |
| Monthly_fixed_charge_system | int | 1 | 1=flat service charge, 2=tiered service charge |
| SC_flat | float | 27.16 | Flat monthly service charge ($) |
| rateStructure | int | 7 | Electricity rate type (1-8, see Section 3.3.7) |
| sellStructure | int | 1 | Grid sell price structure (1=flat, 2=monthly, 3=same as buy) |
| Pbuy_max | float | 50.0 | Maximum grid import power (kW) |
| Psell_max | float | 50.0 | Maximum grid export power (kW) |
| Natural Gas |  |  |  |
| NG_energycontent | float | 10.81 | Energy content of 1 m3 of natural gas (kWh/m3) |
| Furnace_eff | float | 0.94 | Efficiency of natural gas furnace (fraction) |
| NG_unit | str | therms | Unit of NG prices entered: m3, therms, or kwh |
| rateStructure_NG | int | 1 | NG rate type (1-8) |
| SC_flat_NG | float | 18.59 | Flat monthly NG service charge ($) |

# 7. Optimization Algorithms

SAMA includes four metaheuristic optimization algorithms. All four optimize the same objective (NPC, or NPC+LEM when EM=1) over the same 5-variable design space. The choice of algorithm does not affect what is being optimized, only how the search space is explored.

## 7.1 Particle Swarm Optimization (PSO)

PSO simulates a swarm of candidate solutions (particles) that move through the search space. Each particle adjusts its velocity based on its own best-known position and the swarm's global best position. Key parameters:

| Parameter | Default | Description |
| --- | --- | --- |
| w | 1.0 | Inertia weight: controls how much a particle keeps its current velocity |
| wdamp | 0.99 | Inertia weight damping ratio: w is multiplied by wdamp each iteration |
| c1 | 2.0 | Personal learning coefficient: attraction toward particle's own best position |
| c2 | 2.0 | Global learning coefficient: attraction toward swarm's global best position |

## 7.2 Advanced Differential Evolution (ADE)

ADE is a population-based algorithm that creates new candidate solutions by combining existing ones using differential mutation and crossover. SAMA's ADE implementation uses multiple DE strategies with adaptive parameters:

- Five mutation strategies: DE/rand/1, DE/best/1, DE/current-to-best/1, DE/rand/2, DE/best/2

- Adaptive scaling factor F and crossover probability CR, tuned by success rate and iteration progress

- Heterogeneous initialization (random, center-biased, boundary-biased, Sobol-like) for diverse coverage

| Parameter | Default | Description |
| --- | --- | --- |
| F_min / F_max | 0.1 / 0.9 | Minimum/maximum scaling factor for mutation |
| CR_min / CR_max | 0.1 / 0.9 | Minimum/maximum crossover probability |

## 7.3 Improved Artificial Bee Colony (ABC)

ABC mimics the foraging behavior of honey bees. It has three phases: employed bees, onlooker bees, and scout bees, each exploring and exploiting promising regions of the search space. SAMA's ABC uses multi-dimensional perturbations and guided reinitialization of stagnant solutions.

| Parameter | Default | Description |
| --- | --- | --- |
| maxTrials | 15 | Maximum number of trials before a food source is abandoned and re-scouted |
| modification_rate | 0.8 | Probability of modifying multiple dimensions simultaneously |
| initial_search_radius | 0.5 | Search radius at the start of optimization (fraction of variable range) |
| final_search_radius | 0.1 | Search radius at the end of optimization |

## 7.4 Grey Wolf Optimizer (GWO)

GWO mimics the social hierarchy and hunting behavior of grey wolves. Solutions are classified as alpha (best), beta, delta, and omega wolves. The pack converges on prey (optimal solution) guided by alpha, beta, and delta wolves. The parallel variant (ParallelGreyWolfOptimizer) uses Python multiprocessing for faster evaluation when Run_Time > 1.

## 7.5 Choosing an Algorithm

All four algorithms are suitable for this problem. In practice, for the default 5-variable search space with MaxIt=200 and nPop=50, any algorithm will find good solutions. ADE and ABC tend to explore more thoroughly; PSO and GWO converge faster. For a rigorous comparison, use Run_Time > 1 to average across multiple independent runs.

# 8. Energy Management System (EMS)

For each candidate system design, SAMA simulates 8,760 hours of operation using an hourly dispatch model. The EMS decides, for every hour, how much power each component supplies or consumes.

## 8.1 Base Dispatch (EMS.py)

The base EMS uses Numba JIT compilation for fast execution. In each hour it performs marginal-cost-based dispatch:

- Calculate available PV and wind power from irradiance and wind speed.

- Calculate net demand (load minus PV minus wind output).

- If net demand is positive (deficit): draw from battery, then diesel generator, then grid, based on relative hourly cost.

- If net demand is negative (surplus): charge battery, then export to grid if NEM is enabled.

- Track battery SOC, respect SOC_min, SOC_max, and charge/discharge current limits.

- Track any unmet energy (Ens) toward LPSP calculation.

## 8.2 EV Smart Charging and V2X (EMS_EV.py)

When EV = 1, the EMS_EV module adds vehicle-to-home (V2H) and vehicle-to-grid (V2G) capabilities. Key behaviors:

- EV is only available to charge or discharge when it is at home (EV_p = 1, determined by departure time Tout, arrival time Tin, and holiday schedule).

- Departure planning: the EV must reach SOC_dep before Tout each day.

- Lookahead arbitrage: SAMA looks ahead at future grid prices to decide whether to discharge the EV battery now (selling to grid or powering the home) or wait. The decision accounts for battery wear cost and round-trip efficiency losses.

- Safety buffer: a reserve SOC is maintained based on historical departure patterns and load variability.

- V2H discharge is used when it is economically advantageous (current buy price greater than battery wear cost plus discharge losses).

Key EV parameters set in Input_Data.py or the wizard:

| Parameter | Default | Description |
| --- | --- | --- |
| C_ev | 82 kWh | EV battery total capacity |
| SOCe_min / SOCe_max | 0.03 / 0.97 | Minimum and maximum usable SOC fraction |
| Pev_max | 11 kW | Maximum charge and discharge power |
| Range_EV | 468 km | EV full-charge driving range |
| Daily_trip | 68 km | Average daily distance driven |
| SOC_dep | 0.85 | Required SOC at departure time |
| Tin / Tout | 17 / 8 | Arrival hour (17 = 5 PM) and departure hour (8 = 8 AM) |
| n_e | 0.9 | EV battery round-trip charge efficiency |
| L_EV_dis | 400,000 km | EV battery lifetime in total km driven |
| treat_special_days_as_home | false | If true, holidays are treated as home days for EV presence |

## 8.3 Heat Pump Energy Attribution (EMS_HP.py)

When HP = 1, EMS_HP post-processes the dispatch results to attribute the electricity consumed by the heat pump to heating versus cooling loads. Two methods are used: proportional allocation (based on hourly heating and cooling fractions) and marginal-cost-based assignment. This enables detailed multi-fuel cost accounting that separates electricity costs for space heating from space cooling.

# 9. Component Models

## 9.1 Solar PV

Hourly PV power output uses a temperature-dependent efficiency model with NOCT-based cell temperature correction:

- Cell temperature is calculated from ambient temperature, NOCT, and irradiance.

- Module efficiency is adjusted for cell temperature using Tcof (temperature coefficient, default -0.3 %/C).

- Output is multiplied by the derating factor fpv (default 0.9) for real-world losses.

- The default module rated power is Ppv_r = 1 kW; the optimizer returns the number of modules Npv.

## 9.2 Wind Turbine

- Hub-height wind speed is calculated from anemometer height h0 to hub height h_hub using a power-law profile with friction coefficient alfa_wind_turbine (default 0.14).

- Power output uses a piecewise model: zero below cut-in speed (v_cut_in = 2.5 m/s), linear ramp to rated power between cut-in and rated speed (v_rated = 9.5 m/s), rated power between rated and cut-out (v_cut_out = 25 m/s), and zero above cut-out.

- The default rated power is Pwt_r = 1 kW; the optimizer returns the number of turbines Nwt.

## 9.3 Battery Storage

SAMA supports two battery chemistries:

- Li-ion: idealized model with fixed round-trip efficiency ef_bat_Li (default 0.90), maximum charge current Ich_max_Li_ion, maximum discharge current Idch_max_Li_ion, lifetime throughput Q_lifetime_Li (default 3,000 kWh per unit), and nominal capacity Cnom_Li.

- Lead-acid: KiBaM (Kinetic Battery Model) with capacity ratio c and storage rate constant k_lead_acid, maximum charge current Ich_max_leadacid, and lifetime throughput Q_lifetime_leadacid (default 8,000 kWh per unit).

Both types use SOC_min (default 0.1) and SOC_max (default 1.0) to bound the state of charge. The battery rated capacity Cbt_r is derived from the cell voltage and nominal capacity.

## 9.4 Diesel Generator

Fuel consumption is modeled as an affine function of output power: F = a * Prated + b * P_output (L/hr). Parameters a = 0.4388 and b = 0.1097 are default values. The minimum loading ratio LR_DG (default 0.25) prevents the generator from running at very low fractions of rated capacity. Emissions per litre of fuel are set by CO2, CO, NOx, and SO2.

## 9.5 Heat Pump Models

SAMA includes black-box models for two heat pump brands:

- Goodman (BB_HP_Goodman.py): Uses manufacturer data to compute hourly electric consumption for space heating and cooling as a function of outdoor ambient temperature and building thermal load (Hload, Cload). Returns COP curves for heating and cooling modes.

- Bosch (BB_HP_Bosch.py): Same structure as the Goodman model but using Bosch manufacturer data. Select using HP_brand = 'Bosch' or HP_brand = 'Goodman'.

Both models read lookup data from the content/ folder (HP_Bosch/ and HP_Goodman/ subdirectories containing Excel data files).

# 10. Financial Model

## 10.1 Net Present Cost (NPC)

NPC is the total 25-year lifecycle cost of the hybrid energy system in today's dollars. It includes:

- Initial capital cost (I_Cost): PV, WT, DG, battery, inverter, charger, engineering costs, heat pump, EV battery, NEM connection fee, reduced by RE incentives and multiplied by (1 + System_Tax).

- Replacement costs (R_Cost): scheduled component replacements over the project lifetime, discounted to present value.

- O&M costs (MO_Cost): annual operations and maintenance costs for all components, escalated and discounted.

- Fuel cost (C_Fu): diesel generator fuel cost, escalated by C_fuel_adj_rate and discounted.

- Grid electricity cost (Grid_Cost_net): annual grid purchases at tariff rates, escalated by Grid_escalation_rate and discounted. NEM credits and grid credits are subtracted.

- Salvage value: remaining value of components at end of project life.

NPC = (I_Cost + sum(R_Cost) + sum(MO_Cost) + sum(C_Fu) - sum(Salvage)) * (1 + System_Tax) + sum(Grid_Cost_net) + sum(gHeating_Cost)

## 10.2 Levelized Cost of Energy (LCOE)

LCOE = CRF * NPC / E_tot, where CRF is the capital recovery factor derived from the real discount rate ir and project lifetime n, and E_tot is the total discounted energy supplied over the project lifetime (including EV charging and thermal energy from the heat pump).

## 10.3 Levelized Emissions (LEM)

LEM is computed when EM = 1. It is the total lifecycle emissions (from diesel generation and grid-sourced electricity) normalized by total supplied energy. Emission factors CO2, CO, NOx, SO2 (kg/L of diesel) and E_CO2, E_SO2, E_NOx (kg/kWh of grid electricity) are user-specified.

## 10.4 25-Year Cash Flow Projection

The financial model projects cash flows year by year over the n-year project lifetime. Grid electricity prices are escalated using Grid_escalation_rate (array of 25 annual percentages). Natural gas prices are escalated using Grid_escalation_rate_NG. Component replacements are scheduled at multiples of their lifetimes (L_PV, L_WT, L_B, L_I, L_CH, L_HP, L_EV). The Advanced_multi_cashflow module generates multi-run comparison charts when Cash_Flow_adv = 1 and Run_Time > 1.

# 11. Bundled Data Files (content/ folder)

The following data files are bundled with the SAMA package in the sama/content/ directory and can be accessed via get_content_path():

| File | Description |
| --- | --- |
| Eload.csv | Sample hourly electrical load profile (8,760 rows, kW) for a representative building in London, Ontario |
| Eload_daily.csv | Sample daily electrical load profile (24 rows) for generic load scaling |
| METEO.csv | SAM NSRDB meteorological data for London, Ontario. Columns include GHI, DNI, DHI, temperature, wind speed, and pressure |
| Irradiance.csv | Sample hourly plane-of-array (POA) irradiance CSV (8,760 rows, W/m2) for manual G_type=2 input |
| Temperature.csv | Sample hourly ambient temperature CSV (8,760 rows, C) for manual T_type=2 input |
| WSPEED.csv | Sample hourly wind speed CSV (8,760 rows, m/s) for manual WS_type=2 input |
| Battery.csv | Battery lookup data |
| Data.csv | General component reference data |
| Generic_load_JulyP.csv | Normalized generic hourly load profile with July peak; used when load_type = 5/8/9/10 |
| Generic_load_JanuaryP.csv | Normalized generic hourly load profile with January peak; used when load_type = 5/8/9/10 |
| house_load.xlsx | Sample building heating (Hload) and cooling (Cload) thermal load (8,760 rows, kW each) for London, Ontario |
| HP_Bosch/ | Bosch heat pump manufacturer performance data files (6 Excel files and weather data) |
| HP_Goodman/ | Goodman heat pump manufacturer performance data files (8 Excel files and weather data) |

To use your own weather or load data, either replace these files in place or specify alternative file paths in Input_Data.py (e.g. path_Eload, path_G, path_T, path_WS, weather_url) or in the config YAML.

# 12. Input Data Formats

## 12.1 Electrical Load CSV (Eload.csv)

- One column, 8,760 rows (no header).

- Each row is the hourly average electrical load in kW for that hour of the year.

- Hour 0 = midnight on January 1; hour 8,759 = 11 PM on December 31.

## 12.2 METEO.csv (SAM NSRDB format)

- Two header rows followed by hourly data rows (8,760 rows).

- Must include columns: GHI, DNI, DHI, Temperature, Wind Speed, Pressure.

- Downloaded from NSRDB (https://nsrdb.nrel.gov/) or exported from SAM (System Advisor Model).

## 12.3 Plane-of-Array Irradiance CSV (Irradiance.csv)

- One column, 8,760 rows (no header).

- Each row is the hourly POA irradiance in W/m2.

- Used when G_type = 2.

## 12.4 house_load.xlsx (Thermal Load)

- Excel file with at least 3 columns.

- Column index 1 (second column): hourly heating load Hload in kW (8,760 rows).

- Column index 2 (third column): hourly cooling load Cload in kW (8,760 rows).

# 13. Troubleshooting

| Error / Symptom | Cause | Fix |
| --- | --- | --- |
| ModuleNotFoundError: No module named 'sama' | SAMA not installed in current Python environment | Run pip install sama (or pip install -e . from project root) |
| command not found: sama-config | pip install did not add scripts to PATH | On Windows: restart terminal. On Linux/Mac: add ~/.local/bin to PATH |
| BackendUnavailable: Cannot import setuptools.backends.legacy | Old version of setuptools | The pyproject.toml uses setuptools.build_meta. Ensure the correct pyproject.toml is present. |
| ModuleNotFoundError: No module named 'numba' | numba not installed or wrong version for Python 3.13+ | pip install 'numba>=0.60' |
| _tkinter.TclError: no display name | matplotlib trying to open a GUI window on a headless server | Use sama-run --no-gui, or set environment variable MPLBACKEND=Agg |
| FileNotFoundError: Eload.csv not found | Package data not installed correctly | Run pip install --force-reinstall sama |
| Optimization produces 0 kW for all components | All candidate solutions violate constraints (LPSP, RE_min, Budget) | Relax constraints: increase LPSP_max_rate, decrease RE_min_rate, increase Budget or VarMax |
| sama-config freezes or no text appears | Terminal does not support questionary interactive prompts | Try a different terminal: cmd.exe or PowerShell on Windows; bash or zsh on Linux/macOS |
| Very slow optimization (many hours) | numba JIT compilation on first run, or MaxIt/nPop too large | First run includes JIT compilation (expected). Reduce MaxIt or nPop for testing. |
| Results folder is empty after optimization | Output directory not created or permissions issue | Check output_directory in YAML config. Ensure write permissions in that folder. |

# 14. Quick Reference - Minimum Working Example

The following is the minimum set of steps to run SAMA with a fresh pip installation using default bundled data:

- Install SAMA (the SAMA Py Package):

pip install sama

- Open a terminal in an empty working directory.

- Run the configuration wizard:

sama-config

Follow all 18 sections. For a quick test, accept defaults in most sections.

- Run the optimization:

sama-run

- View results in sama_outputs/figs/ and sama_outputs/data/Outputforplotting.csv.

Expected Runtime: The first run always takes longer because numba compiles the EMS functions on first call. After the first run, subsequent runs are significantly faster. A typical run with MaxIt=200, nPop=50 takes 5-30 minutes depending on hardware and which components are enabled.

# 15. Package Module Structure

| Module / File | Description |
| --- | --- |
| sama/core/ |  |
| Input_Data.py | Main parameter class: defines all input data and default values. Singleton pattern: InData = Input_Data() |
| Fitness.py | Fitness evaluation function: calls EMS for each candidate design and returns NPC, LCOE, LEM, LPSP, and constraint penalties |
| sama/cli/ |  |
| wizard.py | Entry point for sama-config command: launches sama_Wizard.py |
| sama_Wizard.py | Interactive 18-section configuration wizard using questionary |
| runner.py | Entry point for sama-run command: loads YAML, applies config, runs selected algorithm |
| config_loader.py | load_config(), apply_config(), save_config(), merge_configs() functions |
| sama/optimizers/ |  |
| swarm.py | Particle Swarm Optimization (Swarm class) |
| AdvancedDifferentialEvolution.py | Advanced Differential Evolution (AdvancedDifferentialEvolution class) |
| ArtificialBeeColony.py | Improved Artificial Bee Colony (ImprovedArtificialBeeColony class) |
| GreyWolfOptimizer.py | Grey Wolf Optimizer (GreyWolfOptimizer and ParallelGreyWolfOptimizer classes) |
| pso.py / ade.py / run_abc.py / gwo.py | Thin wrapper scripts for running each algorithm directly from command line |
| sama/ems/ |  |
| EMS.py | Base hourly dispatch engine (Numba JIT compiled) |
| EMS_EV.py | Extended dispatch engine with EV smart charging and V2X lookahead arbitrage |
| EMS_HP.py | Heat pump energy attribution post-processor |
| sama/models/ |  |
| Battery_Model.py | Battery discharge and degradation models (lead-acid KiBaM and Li-ion) |
| BB_HP_Goodman.py | Goodman air-source heat pump black-box model |
| BB_HP_Bosch.py | Bosch air-source heat pump black-box model |
| sama/pricing/ |  |
| Electricity_Bill_Calculator.py | Monthly and annual billing calculations with escalation and NEM reconciliation |
| calcFlatRate.py | Flat rate tariff calculator |
| calcSeasonalRate.py | Seasonal rate tariff calculator |
| calcMonthlyRate.py | Monthly rate tariff calculator |
| calcTieredRate.py | Tiered rate tariff calculator |
| calcSeasonalTieredRate.py | Seasonal tiered rate calculator |
| calcMonthlyTieredRate.py | Monthly tiered rate calculator (3-tier) |
| calcMonthlyTieredRate4.py | Monthly tiered rate calculator (4-tier, for Enbridge EGD) |
| calcTouRate.py | Time-of-Use (TOU) rate calculator |
| calcULTouRate.py | Ultra-Low TOU (ULO) rate calculator |
| service_charge.py | Monthly utility service charge calculator (flat or tiered) |
| Advanced_multi_cashflow.py | Advanced multi-run cash flow analysis and comparison plots |
| sama/results/ |  |
| Results.py | Gen_Results(): generates all output charts and the Outputforplotting.csv data file |
| sama/utilities/ |  |
| daysInMonth.py | Calendar utility: leap-year aware days-per-month array |
| dataextender.py | Expand monthly averages to 8,760-hour arrays |
| generic_load.py | Generic electrical load profile generator (July-peak and January-peak archetypes) |
| sam_monofacial_poa.py | SAM NSRDB API integration: plane-of-array irradiance and temperature from METEO.csv |
| top_down.py | Top-down PV pricing calculator |
| EV_Presence.py | Determine hourly EV home-presence schedule from departure/arrival times and holidays |
| EV_travel.py | EV travel energy consumption modeling |
| EV_demand_dest.py | EV demand and destination distribution utilities |
| Ev_Battery_Throughput.py | EV battery lifetime throughput calculation based on degradation and km driven |
| sama/content/ |  |
| (data files) | Bundled CSV, Excel, and meteorological data files (see Section 11) |

# 16. License and Citation

## 16.1 License

SAMA is released under the GNU General Public License v3.0 (GPL-3.0). You are free to use, copy, modify, and distribute SAMA under the terms of this license. Any derivative works must also be released under GPL-3.0.

## 16.2 Authors

| Author | Affiliation | ORCID |
| --- | --- | --- |
| Seyyed Ali Sadat | Free Appropriate Sustainability Technology (FAST) Research Group, Western University, London, Ontario, Canada | 0000-0001-9690-4239 |

## 16.3 Citing SAMA

If you use SAMA in academic work, please cite the primary published paper [A1]. A Journal of Open Source Software (JOSS) paper is currently pending submission; once published, that citation should also be included. In the meantime, please cite the GitHub repository and acknowledge the FAST Research Group at Western University.

Primary citation (published):

Sadat, S.A., Takahashi, J. and Pearce, J.M., 2023. A Free and open source microgrid optimization tool: SAMA the solar alone Multi-Objective Advisor. Energy Conversion and Management, 298, p.117686. https://doi.org/10.1016/j.enconman.2023.117686

JOSS paper (pending submission):

Sadat, S.A. and Pearce, J.M. SAMA: Solar Alone Multi-Objective Advisor. Journal of Open Source Software (JOSS). Pending submission.

Additional SAMA publications (see Section 19 for full list):

Sadat & Pearce (2024). Economic grid defection analysis. Solar Energy, 282, 112910.

Sadat & Pearce (2025). Electricity pricing structures in Canada. Renewable Energy, 242, 122456.

Repository: https://github.com/Sas1997/SAMA |  FAST Research Group, Western University  |  GPL-3.0 License

# 17. Technical Framework and Mathematical Formulation

This section presents the mathematical models and equations that underpin SAMA's simulation and optimization engine. The framework described here is drawn from the original SAMA technical documentation [A1] and peer-reviewed literature. All equation numbers correspond to the formulations implemented in the source code.

## 17.1 Particle Swarm Optimization Algorithm

The default optimizer in SAMA is the Particle Swarm Optimizer (PSO), a bio-inspired algorithm that moves a population of candidate solutions (particles) through the search space according to the following velocity and position update equations [A1]:

| Vit+1=W.Vit+c1U1tPb1-Pit+c2U2t(gbt-Pit) | (1) |
| --- | --- |
| Pit+1=Pit+vit+1 | (2) |

In Eq. 1, W is the inertia weight, c1 is the cognitive (personal learning) constant, c2 is the social (global learning) constant, U1 and U2 are random numbers drawn uniformly from [0,1], Pb is the particle's personal best position, and gb is the global best position of the swarm. Eq. 2 updates the particle position using the new velocity. Default PSO parameters in SAMA are W = 1, c1 = 2, c2 = 2, with an inertia weight damping ratio of 0.99 per iteration.

## 17.2 Solar PV Power Output Model

The hourly power output of PV modules (P_PV [kW]) and corresponding PV energy (E_PV [kWh]) are functions of module temperature (T_Module [degC]) and plane-of-array irradiance (POA [W/m2]), calculated as follows [13][14][15]:

| PPV [kW]=NPVfPV×PPVSTCPOAPOASTC1+δPVTModule-TRef EPV=PPVt | (3) |
| --- | --- |

Where N_PV is the optimum number of PV modules (from optimizer), f_PV is the PV derating factor (default 0.9) [7], P_PV_STC [kW] is the rated capacity at Standard Test Conditions (STC: 1000 W/m2 irradiance, 25 degC), POA_STC = 1000 W/m2 is the reference irradiance [10], delta_PV is the temperature coefficient at STC (default -3.7 x 10^-3 /degC) [16], and T_Ref = 25 degC is the STC reference temperature [8].

The module operating temperature T_Module is calculated using the Nominal Operating Cell Temperature (NOCT) model [17]:

| TModule[°C] =TAmb+Tnoct-20800×POA | (4) |
| --- | --- |

Where T_noct [degC] is the nominal operating cell temperature (default 45 degC) [9] and T_Amb [degC] is the ambient temperature from the meteorological dataset.

## 17.3 Battery Storage Model (KiBaM)

SAMA uses the Kinetic Battery Model (KiBaM) [18] to model energy storage, which was originally developed for lead-acid batteries and has been validated for lithium-ion and other chemistries [19][20][21][22]. KiBaM represents the battery as two reservoirs (available and bound energy). The maximum chargeable power P_BT_ch_max_KiBaM [kW] is given by [18]:

| PBT,chmax, KiBaM=kcQMax+kQ1e-kt+Qkc1-e-kt1-e-kt+c(kΔt-1+e-kt) ;EBT,chmax, KiBaM=PBT,chmax, KiBaMt | (5) |
| --- | --- |

Where Q1 is the available energy at the start of the time-step [kWh], Q_max is the total storage capacity [kWh], Q is total energy in storage at the start of the time-step [kWh], c is the storage capacity ratio, k is the storage rate constant [h^-1], and t is the simulation time-step. Following HOMER Pro [23], SAMA also applies two additional charging limits based on the maximum charge rate (alpha [A/Ah]) and maximum current (I_max):

| PBT,chmax, mcr=(1-e-αt)(Qmax-Q)tEBT,chmax, mcr=PBT,chmax, mcrt | (6) |
| --- | --- |
| PBT,chmax, mcc=NBTImaxVnom1000EBT,chmax, mcc=PBT,chmax, mcct | (7) |

The actual maximum charge power is the minimum of the three limits divided by round-trip efficiency:

| PBT,chmax=MinPBT,chmax, KiBaMPBT,chmax, mcr,PBT,chmax, mccηBT EBT,chmax=PBT,chmaxt | (8) |
| --- | --- |

The maximum discharge power is calculated from [18]:

| PBT,dchmax, KiBaM=PBT,dchmax=kQ1e-t+Qkc1-e-kt1-e-kt+c(kΔt-1+e-kt) EBT,dchmax=PBT,dchmaxt | (9) |
| --- | --- |

The battery wear cost (Cost_BT_Wear [$/kWh]) represents the marginal cost of each kWh cycled through the battery [24][25][26]:

| CostBTWear=RBT×NBTtotalNbat×Qlifetime×ηBT | (10) |
| --- | --- |

Where R_BT is the replacement cost per kWh, N_BT_total is the total battery storage capacity, N_bat is the number of battery units, and Q_lifetime is the lifetime throughput in kWh.

Battery energy for the next time-step is updated as [A1]:

| EBTt+1=(1-δ)×EBTt+ηBT×EBTch(t)-EBTdch(t)ηBT | (37) |
| --- | --- |

Where delta is the hourly self-discharge rate.

## 17.4 Backup (Diesel or Gasoline) Generator Model

The Backup generator (BG) is modeled as a backup dispatchable source. The total hourly operating cost of the DG is calculated from its fuel consumption using a linear fuel curve [26][27][28]:

| CostDG=b×PDGNominal×Costfuel+RDG×PDGNominalTLDG+MODG+a×Costfuel | (11) |
| --- | --- |

Where a [L/hr/kW output] and b [L/hr/kW rated] are the slope and intercept of the fuel consumption curve (defaults a = 0.2730, b = 0.0330 from [11]), P_DG_Nominal [kW] is the total rated DG power, Cost_fuel [$/L] is the diesel fuel cost, R_DG [$/kW] is the replacement cost, TL_DG [hours] is the DG lifetime, and MO_DG [$/op.h] is the operations and maintenance cost.

## 17.5 Wind Turbine Model

Hub-height wind speed is extrapolated from anemometer height h0 to hub height h_hub using the power-law wind shear model:

| V_hub = V_anemometer * (h_hub / h0)^alpha_wind_turbine | (WT) |
| --- | --- |

Where alpha_wind_turbine is the friction coefficient (default 0.14 for open terrain). Wind turbine output follows a piecewise power curve: zero below cut-in speed (v_cut_in = 2.5 m/s), linear ramp from cut-in to rated speed (v_rated = 9.5 m/s), constant rated power from rated to cut-out speed (v_cut_out = 25 m/s), and zero above cut-out.

## 17.6 Energy Management Strategy (EMS)

SAMA implements an advanced load-following dispatch strategy [29][30] that operates over all 8,760 hours of the year. In each time-step t, when renewable generation (E_RE) exceeds load demand (E_load), excess energy is used to charge the battery and sell to the grid:

| EBTcht=minEBTempty,EREt-Eloadtηinvand EBTch(t)=min (EBTcht, EBT,chmax) | (12) |
| --- | --- |
| EBTempty=EBTmax-EBTηBT | (13) |
| EACsur=ηinv×(ERE(t)-EBTch(t))-Eload(t) | (14) |
| Esellt=minEACsur,Esell,max and Esellt=min⁡(max0,Einvmax-Eload),Esellt) | (15) |

When load exceeds renewable generation, SAMA selects the dispatch order among battery, DG, and grid based on marginal cost comparison. Six prioritization scenarios are defined based on the relative values of the grid buy price (C_buy), DG cost (Cost_DG), and battery wear cost (Cost_BT_Wear):

Scenario 1 (Grid, DG, Battery): C_buy <= Cost_DG and Cost_DG <= Cost_BT_Wear

Scenario 2 (Grid, Battery, DG): C_buy <= Cost_BT_Wear and Cost_BT_Wear < Cost_DG

Scenario 3 (DG, Grid, Battery): Cost_DG < C_buy and C_buy <= Cost_BT_Wear

Scenario 4 (DG, Battery, Grid): Cost_DG < Cost_BT_Wear and Cost_BT_Wear < C_buy

Scenario 5 (Battery, DG, Grid): Cost_BT_Wear < Cost_DG and Cost_DG < C_buy

Scenario 6 (Battery, Grid, DG): All remaining cases

The loss of power supply probability (LPSP) constraint tracks unmet energy Ens(t) across the year:

| LPSP=t8760Ens(t)t8760Eload(t) | (53) |
| --- | --- |

The renewable fraction (RF) constraint ensures the minimum renewable share is met [6]:

| RF=1-t8760Enon renewable(t)t8760(Eloadt-ENS(t)) | (54) |
| --- | --- |

## 17.7 Economic Model

The real discount rate i is derived from the nominal discount rate i' and expected inflation rate f [A1]:

| i=i'-f1+f | (38) |
| --- | --- |

The Net Present Cost (NPC) is the total lifecycle cost of the system discounted to present value [31]:

| NPC=CI+CR+CMO+CF-CS+CG1+in | (39) |
| --- | --- |

The initial investment cost C_I includes all components [5]:

| CI=CPVPPVtotal+ CDGPDGtotal+CBTNtotalBT+CInvPInv+CCH+CEng | (40) |
| --- | --- |

Engineering costs C_Eng aggregate installation, overhead, sales and marketing, permitting and inspection, electrical and structural balance-of-system, supply chain, profit, and sales tax from NREL PV cost benchmarks [3]. Replacement, O&M, and fuel costs are:

| CR= CPVRPPVtotal+ CDGRPDGtotal+CBTRNtotalBT+CInvRPInv+CCHR | (41) |
| --- | --- |
| CMO=CPVMOPPVtotal+ CDGMOPDGtotal+CBTMONtotalBT+CInvMOPInv+CCHMO | (42) |
| CF=08760Fuel cost per literq | (43) |

Salvage values for each component are calculated based on remaining lifetime [32]:

| CS=SPV + SDG + SBT + Sinv + SCH in which Scomp=Rcompnremncomp | (44) |
| --- | --- |

Total grid cost over the project lifetime includes fixed annual charges, monthly service charges, and variable hourly buy/sell costs [A1]:

| CG=CGridFixed, Annual+012CGridFixed, monthly+08760CbuyPbuy-08760CsellPsell | (45) |
| --- | --- |

The Capital Recovery Factor (CRF) annualizes the NPC for LCOE calculation [33]:

| CRFi,N=i1+in1+in-1 | (46) |
| --- | --- |

The Levelized Cost of Energy (LCOE) is then [33]:

| LCOE=CRFNPC08760(Eload-Ens+Psell) | (47) |
| --- | --- |

When multi-objective optimization is enabled (EM = 1), SAMA also minimizes Levelized Emission (LE), defined similarly to [34]:

| LE=08760Non Grid Emissions(t)+08760GridEmissions(t)08760Eload (t)-Ens(t) | (49) |
| --- | --- |

DG fuel consumption and emissions are calculated from the linear fuel curve [26]:

| q=aPDG+bPDGNominal | (51) |
| --- | --- |

Grid emissions are proportional to electricity imported:

| Grid Emissions =EbuyEFGrid | (52) |
| --- | --- |

The multi-objective function combines NPC and LE, with penalties applied when constraints are violated [A1]:

| Z=NPC+EMLE+penalties | (55) |
| --- | --- |

Table 2 summarizes the penalty conditions applied to infeasible candidate solutions:

| Penalty Condition | Reason |
| --- | --- |
| DC to AC ratio > 1.99 * (P_Inv + P_DG + P_buy_max) | DC to AC ratio must not exceed 2 for a feasible inverter design |
| LPSP > LPSP_max | System does not supply sufficient power to meet reliability constraint |
| RF < RE_min | Renewable fraction is below the user-specified minimum |
| I_Cost > Budget | Initial capital cost exceeds the user-specified budget limit |

Table 2. Penalty conditions in SAMA optimization (from [A1]).

## 17.8 Validation Against HOMER Pro

SAMA results have been cross-validated against HOMER Pro across two geographically distinct U.S. locations: Sacramento, California and New Bern, North Carolina [A1]. Validation used identical input data (same load profiles from OpenEI [2], meteorological data from NSRDB [1], and equivalent pricing). The PSO optimizer was run with MaxIt = 101 and nPop = 100, repeated 10 times to assess convergence.

Convergence was achieved by iteration 70 on the first run and as early as iteration 20 on subsequent runs, demonstrating consistent optimization behavior. Final system configurations, LCOE, NPC, energy distribution, and battery state-of-charge profiles from SAMA showed close agreement with HOMER Pro across all tested system types (PV+Grid, PV+BT+Grid, PV+BT+DG+Grid, PV+BT+DG, PV+BT).

Minor differences between SAMA and HOMER Pro outputs are attributable to differences in dispatch strategy implementation, pricing methodology rounding, and the fact that SAMA's load-following EMS was developed independently with a stronger emphasis on minimizing power shortages. A key advantage of SAMA is its novel top-down pricing methodology [A1], which is not available in HOMER Pro and allows sensitivity analysis on total PV system cost per kW without requiring itemized component cost inputs.

For the off-grid PV+DG+Battery configuration in Sacramento, the optimizer found that maintaining a grid connection (LCOE = $0.0838/kWh) is significantly more economical than full grid defection ($0.3674/kWh). Similar conclusions apply to New Bern. This capability makes SAMA particularly useful for grid-defection analysis [A2].

# 18. References

The following references are cited in this documentation and in the SAMA technical framework described in Section 17. For access to SAMA-related publications, contact the FAST Research Group at Western University (see Section 19).

[1] National Solar Radiation Database (NSRDB). "Solar Resource Maps and Data." https://nsrdb.nrel.gov (accessed Nov. 07, 2022).

[2] Open Energy Data Initiative (OEDI), OpenEI. https://data.openei.org/ (accessed Apr. 14, 2023).

[3] V. Ramasamy et al., "US Solar Photovoltaic System and Energy Storage Cost Benchmarks, With Minimum Sustainable Price Analysis: Q1 2022," National Renewable Energy Lab. (NREL), Golden, CO (United States), 2022.

[4] Westinghouse. "iGen2200 Inverter Generator." https://westinghouseoutdoorpower.com (accessed Jan. 12, 2023).

[5] A.L. Bukar, C.W. Tan, L.K. Yiew, R. Ayop, and W.-S. Tan, "A rule-based energy management scheme for long-term optimal capacity planning of grid-independent microgrid optimized by multi-objective grasshopper optimization algorithm," Energy Convers. Manag., vol. 221, p. 113161, Oct. 2020.

[6] J. Hirvonen and K. Siren, "A novel fully electrified solar heating system with a high renewable fraction - Optimal designs for a high latitude community," Renew. Energy, vol. 127, pp. 298-309, Nov. 2018.

[7] C.P. Cameron, W.E. Boyson, and D.M. Riley, "Comparison of PV system performance-model predictions with measured PV system performance," in 2008 33rd IEEE PVSC, May 2008, pp. 1-6.

[8] G. Blaesser and E. Rossi, "Extrapolation of outdoor measurements of PV array I-V characteristics to standard test conditions," Sol. Cells, vol. 25, no. 2, pp. 91-96, Nov. 1988.

[9] V. Sun, A. Asanakham, T. Deethayat, and T. Kiatsiriroat, "Evaluation of nominal operating cell temperature (NOCT) of glazed photovoltaic thermal module," Case Stud. Therm. Eng., vol. 28, p. 101361, Dec. 2021.

[10] L. Dunn, M. Gostein, and K. Emery, "Comparison of pyranometers vs. PV reference cells for evaluation of PV array performance," in 2012 38th IEEE PVSC, Jun. 2012, pp. 002899-002904.

[11] M.R. Elkadeem et al., "Feasibility analysis and optimization of an energy-water-heat nexus supplied by an autonomous hybrid renewable power generation system," Desalination, vol. 504, p. 114952, May 2021.

[12] A.F. Guven, N. Yorukeren, and M.M. Samy, "Design optimization of a stand-alone green energy system of university campus based on Jaya-Harmony Search and Ant Colony Optimization algorithms approaches," Energy, vol. 253, p. 124089, Aug. 2022.

[13] A. Seifi et al., "An optimal programming among renewable energy resources and storage devices for responsive load integration," J. Energy Storage, vol. 27, p. 101126, Feb. 2020.

[14] S.A. Sadat, B. Hoex, and J.M. Pearce, "A Review of the Effects of Haze on Solar Photovoltaic Performance," Renew. Sustain. Energy Rev., vol. 167, p. 112796, Oct. 2022. https://doi.org/10.1016/j.rser.2022.112796

[15] A.H. Mondal and M. Denich, "Hybrid systems for decentralized power generation in Bangladesh," Energy Sustain. Dev., vol. 14, no. 1, pp. 48-55, Mar. 2010.

[16] A. Maleki and F. Pourfayaz, "Optimal sizing of autonomous hybrid photovoltaic/wind/battery power system with LPSP technology," Sol. Energy, vol. 115, pp. 471-483, May 2015.

[17] H. Lan et al., "Optimal sizing of hybrid PV/diesel/battery in ship power system," Appl. Energy, vol. 158, pp. 26-34, Nov. 2015.

[18] J.F. Manwell and J.G. McGowan, "Lead acid battery storage model for hybrid energy systems," Sol. Energy, vol. 50, no. 5, pp. 399-405, May 1993.

[19] G.P. Fenner et al., "Comprehensive Model for Real Battery Simulation Responsive to Variable Load," Energies, vol. 14, no. 11, 2021.

[20] L.M. Rodrigues et al., "A Temperature-Dependent Battery Model for Wireless Sensor Networks," Sensors, vol. 17, no. 2, 2017.

[21] Y. Zhao et al., "An iterative learning approach to identify fractional order KiBaM model," IEEE/CAA J. Autom. Sin., vol. 4, no. 2, pp. 322-331, 2017.

[22] L.M. Rodrigues et al., "An analytical model to estimate the state of charge and lifetime for batteries with energy harvesting capabilities," Int. J. Energy Res., vol. 44, no. 7, pp. 5243-5258, 2020.

[23] HOMER Energy. "How HOMER Calculates the Maximum Battery Charge Power." https://www.homerenergy.com/products/pro/docs/latest/ (accessed Nov. 07, 2022).

[24] F.H. Jufri et al., "Optimal Battery Energy Storage Dispatch Strategy for Small-Scale Isolated Hybrid Renewable Energy System," Energies, vol. 14, no. 11, 2021.

[25] M. Ashari, C.V. Nayar, and W.W.L. Keerthipala, "Optimum operation strategy and economic analysis of a photovoltaic-diesel-battery-mains hybrid UPS," Renew. Energy, vol. 22, no. 1, pp. 247-254, Jan. 2001.

[26] T. Lambert, P. Gilman, and P. Lilienthal, "Micropower system modeling with HOMER," Integr. Altern. Sources Energy, vol. 1, no. 1, pp. 379-385, 2006.

[27] H. Suryoatmojo, A.A. Elbaset, and T. Hiyama, "Economic and reliability evaluation of wind-Diesel-Battery system for isolated island considering CO2 emission," IEEJ Trans. Power Energy, vol. 129, no. 8, pp. 1000-1008, 2009.

[28] A. Yahiaoui et al., "Grey wolf optimizer for optimal design of hybrid renewable energy system PV-Diesel Generator-Battery," Sol. Energy, vol. 158, pp. 941-951, Dec. 2017.

[29] C.D. Barley, "Modeling and optimization of dispatch strategies for remote hybrid power systems," Ph.D. dissertation, Colorado State University, 1996.

[30] C.D. Barley and C.B. Winn, "Optimal dispatch strategy in remote hybrid power systems," Sol. Energy, vol. 58, no. 4, pp. 165-179, Oct. 1996.

[31] H. Song et al., "A novel hybrid energy system for hydrogen production and storage in a depleted oil reservoir," Int. J. Hydrog. Energy, vol. 46, no. 34, pp. 18020-18031, May 2021.

[32] HOMER Energy. "Salvage Value." https://www.homerenergy.com/products/pro/docs/3.11/salvage_value.html (accessed Jun. 05, 2023).

[33] D.P. Clarke, Y.M. Al-Abdeli, and G. Kothapalli, "Multi-objective optimisation of renewable hybrid energy systems with desalination," Energy, vol. 88, pp. 457-468, Aug. 2015.

[34] A. Parlikar et al., "The carbon footprint of island grids with lithium-ion battery systems: An analysis based on levelized emissions of energy supply," Renew. Sustain. Energy Rev., vol. 149, p. 111353, Oct. 2021.

# 19. SAMA Publications and Further Reading

The following peer-reviewed publications describe SAMA's development, validation, and application across a range of techno-economic energy studies. Users who wish to cite SAMA in academic work should reference these papers. If you need copies of any of these publications, please contact the developers at the FAST Research Group, Western University.

[A1] Sadat, S.A., Takahashi, J. and Pearce, J.M., 2023. A Free and open-source microgrid optimization tool: SAMA the solar alone Multi-Objective Advisor. Energy Conversion and Management, 298, p.117686. https://doi.org/10.1016/j.enconman.2023.117686

[A2] Sadat, S.A. and Pearce, J.M., 2024. The threat of economic grid defection in the US with solar photovoltaic, battery and generator hybrid systems. Solar Energy, 282, p.112910. https://doi.org/10.1016/j.solener.2024.112910

[A3] Sadat, S.A. and Pearce, J.M., 2025. Techno-economic evaluation of electricity pricing structures on photovoltaic and photovoltaic-battery hybrid systems in Canada. Renewable Energy, 242, p.122456. https://doi.org/10.1016/j.renene.2025.122456

[A4] Sadat, S.A., Mittal, K. and Pearce, J.M., 2025. Using investments in solar photovoltaics as inflation hedges. Energies, 18(4), p.890. https://doi.org/10.3390/en18040890

[A5] Groza, J.M., Sadat, S.A., Hayibo, K.S. and Pearce, J.M., 2024. Using a ledger to facilitate autonomous peer-to-peer virtual net metering of solar photovoltaic distributed generation. Solar Energy Advances, 4, p.100064. https://doi.org/10.1016/j.seja.2024.100064

[A6] Sadat, S.A., Lemieux, J.E.B., and Pearce, J.M., 2026. Shifting Subsidies: Implications of Redirecting Alberta's Oil and Gas Government Support to Solar Power. Energies, 19(4), 972. https://doi.org/10.3390/en19040972

[A7] Sadat, S.A., Hoex, B. and Pearce, J.M., 2022. A review of the effects of haze on solar photovoltaic performance. Renewable and Sustainable Energy Reviews, 167, p.112796. https://doi.org/10.1016/j.rser.2022.112796

For the most up-to-date list of SAMA publications and to request preprints or reprints, visit the FAST Research Group GitHub at https://github.com/Sas1997/SAMA or contact the corresponding author directly.

# Appendix A: Where to Find Input Data for SAMA

This appendix provides practical guidance on where to obtain real-world data for each major input category in SAMA. For each parameter group, the purpose, typical values, and recommended data sources are described so that you can replace the bundled defaults with values appropriate for your specific project location and system.

## A.1 Financial and Economic Parameters

Nominal Discount Rate: Used to discount future cash flows to present value in NPC calculations. Check the Federal Reserve primary credit rate at federalreserve.gov or financial data sites such as YCharts for U.S. projects. For non-U.S. projects, use your local central bank rate. Typical values are 2 to 5%. Input as a percentage (e.g., 4 for 4%).

Expected Inflation Rate: Accounts for anticipated price rises over the project lifetime. For U.S. projects, refer to the Bureau of Labor Statistics CPI reports at bls.gov, or check Federal Reserve projections at federalreserve.gov. For other countries, use national statistics agencies. Input as a percentage (e.g., 3 for 3%).

RE Incentives / Federal Tax Credit: In the United States, the Investment Tax Credit (ITC) provides a 30% credit for solar under the Inflation Reduction Act. Check the Database of State Incentives for Renewables and Efficiency (DSIRE) at dsireusa.org for current federal and state programs. For non-U.S. projects, consult your national energy ministry or IRENA incentive databases.

## A.2 System Constraints

LPSP (Loss of Power Supply Probability): Set to 0% for critical loads (medical, emergency). For non-critical off-grid applications, 1 to 5% is typical. IEEE and HOMER Energy publications provide benchmark values for different application types.

Minimum Renewable Energy Fraction: Align with local Renewable Portfolio Standards (RPS) or your project sustainability target. DSIRE at dsireusa.org lists U.S. state RPS requirements. IRENA reports provide international benchmarks.

Net Metering Cap (kW): Check your utility net metering policy at dsireusa.org. Common U.S. residential caps range from 10 to 100 kW. Non-U.S. users should check local grid interconnection regulations.

Rooftop Area Limit: Measure usable roof area using Google Earth, satellite imagery, or a professional site survey. Subtract areas for chimneys, vents, setbacks (typically 3 feet from roof edges), and shading. As of 2025, high-efficiency monocrystalline panels require approximately 7 to 9 m2 per kW installed including spacing.

## A.3 Weather and Meteorological Data (METEO.csv)

The bundled METEO.csv is for London, Ontario. For any other location, download a new file. The file must contain GHI, DNI, DHI, Temperature, Wind Speed, and Pressure columns with two header rows followed by 8,760 hourly rows.

Primary source: National Solar Radiation Database (NSRDB) at nsrdb.nrel.gov. Provides free hourly weather data for the U.S., Canada, and many other countries. Select the SAM/CSV export format. This is the recommended source for SAMA.

Alternative: NASA POWER at power.larc.nasa.gov provides global coverage for any coordinates. Export to CSV and reformat columns to match the METEO.csv structure.

POA irradiance (for G_type = 2): Use PVWatts at pvwatts.nrel.gov or SAM at sam.nrel.gov to compute hourly plane-of-array irradiance from your tilt and azimuth. Save as Irradiance.csv (one column, 8,760 rows, no header, W/m2).

Tip: When downloading from NSRDB, select the Typical Meteorological Year (TMY) dataset rather than a single calendar year. TMY data is specifically designed for long-term energy system analysis.

## A.4 Electrical Load Data (Eload.csv)

Eload.csv requires one column of 8,760 hourly load values in kW with no header. The bundled file is for a London, Ontario residential building. Options to replace it:

Smart meter data: Most utilities provide 15-minute or hourly interval data through their customer portal (Green Button data in North America). Download and aggregate to hourly kW values.

From bills: Use SAMA load modes 3 to 8 (monthly or annual average inputs) to let SAMA generate a synthetic 8,760-hour profile from your monthly kWh totals.

Open databases: OpenEI at openei.org provides representative commercial and residential load profiles by building type and U.S. climate zone. DOE Commercial Reference Building models are also freely available.

## A.5 Thermal Load Data (house_load.xlsx) for Heat Pump

house_load.xlsx must have hourly heating load (Hload) in column 2 and cooling load (Cload) in column 3, 8,760 rows each, in kW. Options:

EnergyPlus: Free DOE building simulation software at energyplus.net. Build a model with local weather and export hourly heating/cooling loads. Most accurate approach.

RETScreen: Free software from Natural Resources Canada at nrcan.gc.ca. Provides monthly heating and cooling demand that can be distributed hourly.

Manual entry: In sama-config, set Tload_type to 2 or higher and enter monthly average loads. SAMA generates a synthetic hourly profile.

## A.6 Solar PV Parameters

Module efficiency and temperature coefficient: From the manufacturer datasheet. Use EnergySage (energysage.com) or the California Energy Commission (CEC) equipment list to compare modules. Leading 2025 monocrystalline modules range from 20 to 25% efficiency.

Capital and O&M costs: NREL cost benchmark reports at nrel.gov, SEIA quarterly market reports at seia.org, or installer quotes via EnergySage. NREL reports residential PV system costs of USD 2,500 to 3,500 per kW installed and O&M of USD 20 to 40 per kW per year for 2025.

RE incentives: Federal ITC (30%) and state programs at dsireusa.org. Factor incentives into capital cost or use the RE_incentives_rate field.

Azimuth and tilt: For the Northern Hemisphere, south-facing (180 degrees) at a tilt equal to your site latitude maximizes annual yield. Use PVWatts (pvwatts.nrel.gov) to optimize for your specific location.

## A.7 Wind Turbine Parameters

Wind speed data: Included in the METEO.csv from NSRDB. NASA POWER (power.larc.nasa.gov) also provides global wind data at multiple heights.

Turbine specs (hub height, cut-in/rated/cut-out speeds): From the manufacturer datasheet. Bergey Windpower, Northern Power Systems, and other small turbine manufacturers publish detailed performance curves. Typical small turbine values: cut-in 2.5 to 3.5 m/s, rated speed 9 to 13 m/s, cut-out 25 m/s.

Friction coefficient (wind shear exponent): Use 0.14 for open flat terrain, 0.20 for suburban, 0.30 for forested or urban. WindExchange at windexchange.energy.gov provides terrain roughness guidance.

Capital costs: NREL Distributed Wind Market Report at nrel.gov/wind. Small residential turbines (under 100 kW) cost approximately USD 4,000 to 8,000 per kW installed in 2025.

## A.8 Battery Storage Parameters

Capital cost: NREL Annual Technology Baseline at nrel.gov, SEIA reports, EnergySage battery quotes. Li-ion installed costs average USD 200 to 400 per kWh in the U.S. for 2025. Lead-acid is lower cost but requires more frequent replacement.

Round-trip efficiency and SOC limits: From manufacturer datasheet. Li-ion: 90 to 95% round-trip efficiency. Lead-acid: 70 to 85%. Battery University at batteryuniversity.com provides practical guidance. Most manufacturers recommend keeping lead-acid above 50% SOC.

Lifetime throughput: From manufacturer cycle life tables. NREL and PNNL publish battery degradation models for Li-ion. Combine rated cycle count with depth of discharge per cycle to estimate total kWh throughput.

## A.9 Diesel Generator Parameters

Fuel cost per liter: U.S. EIA weekly petroleum reports at eia.gov/petroleum, GasBuddy at gasbuddy.com, or GlobalPetrolPrices at globalpetrolprices.com. Convert from dollars per gallon to dollars per liter by dividing by 3.785.

Fuel curve coefficients (a and b): From the manufacturer fuel consumption data sheet. HOMER Energy documentation and Woodstock Power/Generator Source websites publish consumption curves for common generator sizes. Typical: a = 0.24 to 0.44 L/hr per kW output, b = 0.01 to 0.11 L/hr per kW rated.

CO2 emissions: U.S. EIA default is 2.68 kg CO2 per liter of diesel. The Climate Registry at theclimateregistry.org and EPA emission factor documents provide updated values.

## A.10 Heat Pump Parameters

SAMA includes built-in performance models for Bosch and Goodman brand heat pumps. Select the brand that most closely matches your planned installation. If using a different brand, choose the model with the most similar efficiency ratings.

Rated capacity: Determined by Manual J heating/cooling load calculations. HVAC contractors perform Manual J per ACCA standards. The required capacity is typically 15 to 30 BTU/hr per square foot of floor area depending on climate.

Capital cost: EnergySage, Angi, and Home Depot provide current quotes. For 2025, mini-split systems cost approximately USD 3,000 to 8,000 installed for a 12,000 BTU unit. Normalize to dollars per 1,000 BTU/hr for SAMA input. Federal IRA rebates (up to USD 2,000) and state incentives at dsireusa.org can reduce effective cost.

## A.11 Electric Vehicle Parameters

Battery capacity, range, and charging specs: EV-Database.org, InsideEVs (insideevs.com), or the U.S. EPA at fueleconomy.gov. Manufacturer specification sheets provide usable battery capacity, AC charging power, and efficiency ratings.

Battery replacement cost: Recurrent Auto (recurrentauto.com) tracks real-world EV battery health and replacement cost estimates. Manufacturer warranty documentation covers replacement terms. As of 2025, battery costs are approximately USD 100 to 150 per kWh.

Daily trip distance: From your own odometer records or U.S. DOT National Household Travel Survey (NHTS) at nhts.ornl.gov. U.S. average daily vehicle travel is approximately 50 to 65 km.

Charging schedule: Based on your own commuting pattern. For default values, use the U.S. DOT NHTS or INRIX traffic data for regional averages.

## A.12 Grid and Utility Rate Parameters

Grid rate parameters have the largest impact on SAMA economic results. Always use your actual utility tariff rather than national averages.

Electricity rate schedule: Download from your utility website or OpenEI Utility Rate Database at openei.org/apps/USURDB. Covers most U.S. and many Canadian utilities. Identify your plan type (flat, tiered, TOU, ULO) and enter the matching structure in SAMA.

Current electricity prices: From your most recent utility bill. U.S. EIA publishes monthly state average prices at eia.gov/electricity. Canadian prices from the Canadian Energy Regulator at cer-rec.gc.ca.

Electricity price escalation: Review historical rate filings on your state Public Utility Commission (PUC) website. EIA Annual Energy Outlook provides regional long-term projections. U.S. average is 2 to 3% per year historically.

Net metering policy: DSIRE at dsireusa.org for state rules. Your utility interconnection agreement specifies the export compensation rate. Some states have moved to reduced net metering rates (e.g., California NEM 3.0).

Monthly fixed charge and taxes: From your utility bill line items. Monthly service charge is listed as a fixed dollar amount. Sales tax and per-kWh surcharges are itemized separately.

Natural gas rates: From your gas utility bill or U.S. EIA natural gas reports at eia.gov/naturalgas. Canadian rates from provincial utility regulators. Identify the price per m3 or per therm and rate structure.

Tip: For TOU or ULO rate structures, contact your utility customer service or visit their website for the exact on-peak, mid-peak, and off-peak hour windows. These vary significantly by utility and season.

## A.13 Summary of Recommended Data Sources

| Input Category | Recommended Data Source |
| --- | --- |
| Weather (METEO.csv) | NSRDB (nsrdb.nrel.gov), NASA POWER (power.larc.nasa.gov) |
| POA Irradiance (Irradiance.csv) | PVWatts (pvwatts.nrel.gov), SAM (sam.nrel.gov) |
| Electrical load (Eload.csv) | Utility smart meter / Green Button data, OpenEI load profiles |
| Thermal load (house_load.xlsx) | EnergyPlus (energyplus.net), RETScreen (nrcan.gc.ca) |
| PV module specs and costs | Manufacturer datasheet, EnergySage, NREL cost benchmarks |
| PV incentives | DSIRE (dsireusa.org), IRS ITC documentation |
| Wind turbine specs and costs | Manufacturer datasheet, NREL Distributed Wind Market Report |
| Battery costs | NREL Annual Technology Baseline, SEIA reports, EnergySage |
| Battery specs | Manufacturer datasheet, Battery University (batteryuniversity.com) |
| Diesel fuel cost | EIA weekly petroleum (eia.gov), GasBuddy, GlobalPetrolPrices |
| Diesel emissions factors | U.S. EPA emission factors, The Climate Registry (theclimateregistry.org) |
| Heat pump costs | EnergySage, Angi, DSIRE for IRA rebates |
| EV specs and battery cost | EV-Database.org, InsideEVs, fueleconomy.gov, Recurrent Auto |
| Electricity rates | Your utility bill, OpenEI USURDB (openei.org), EIA |
| Net metering policy | DSIRE (dsireusa.org), utility interconnection agreement |
| Discount rate | Federal Reserve (federalreserve.gov), local central bank |
| Inflation rate | BLS CPI (bls.gov), Federal Reserve projections |
| Natural gas rates | Gas utility bill, EIA natural gas (eia.gov/naturalgas) |